# Appendix III (informative):
ITU-T G.8052 Information Model

This informative annex contains the result of the generation of the NFV Information model using gendoc.

## Class Diagrams

Qualified Name: G.8052_v2.07-model::Class Diagrams

### Diagrams

Figure 1 Diagram G8052_Client and Server Layers of Ethernet Technology Class Diagram

Figure 1 Diagram G8052_ETH CTP Pool Class Diagram

Figure 1 Diagram G8052_TTP Inheritance Class Diagram

Figure 1 Diagram G8052_CTP Inheritance Class Diagram

Figure 1 Diagram G8052_Threshold Profile Control

Figure 1 Diagram G8052_MIP Class Diagram

Figure 1 Diagram G8052_Operation Exceptions Class Diagram

Figure 1 Diagram G8052_Connection Fragment Class Diagram

Figure 1 Diagram G8052_PhysicalSubsytem Class Diagram

Figure 1 Diagram G8052_LAG Management Class Diagram

Figure 1 Diagram G8052_Ethernet Multiplexing Configuration Class Diagram

Figure 1 Diagram G8052_Ethernet Adaptation Modelling Class Diagram

Figure 1 Diagram G8052_On Demand Measurement Class Diagram

Figure 1 Diagram G8052_TCS Class Diagram

Figure 1 Diagram G8052_Pro Active Measurement Class Diagram

Figure 1 Diagram G8052_MEP Class Diagram

Figure 1 Diagram G8052_Ring Protection Class Diagram

Figure 1 Diagram G8052_MCC Class Diagram

Figure 1 Diagram G8052_MEP Class Diagram Lower Part

Figure 1 Diagram G8052_Connection Fragment Linear Protection Part Class Diagram

Figure 1 Diagram G8052_Identifers

Figure 1 Diagram G8052_Containment Tree

Figure 1 Diagram G8052_Containment Tree - Measurement Job

Figure 1 Diagram G8052_ETH Spec

## Imported Data Types

Qualified Name: G.8052_v2.07-model::Imported Data Types

### Datatypes

#### GeneralizedTime primitive type

Qualified Name: G.8052_v2.07-model::Imported Data Types::GeneralizedTime

#### Name primitive type

Qualified Name: G.8052_v2.07-model::Imported Data Types::Name

#### NameAndValue datatype

Qualified Name: G.8052_v2.07-model::Imported Data Types::NameAndValue

Description: A scoped name-value pair

Table 1 Attributes for Data Type NameAndValue

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| valueName | String | 0..1 | The name of the value. The value need not have a name. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| value | String | 1 | The value | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _nameAndValueAuthorityRef |  | 0..1 | The authority that defines the named value. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _globalClassRef |  | 0..1 | The scope of the name uniqueness | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _localClassRef |  | 0..1 | The scope of the name uniqueness | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ObjectIdentifier primitive type

Qualified Name: G.8052_v2.07-model::Imported Data Types::ObjectIdentifier

#### ObjectInstance primitive type

Qualified Name: G.8052_v2.07-model::Imported Data Types::ObjectInstance

#### TimePeriod primitive type

Qualified Name: G.8052_v2.07-model::Imported Data Types::TimePeriod

Description: This primitive Type is imported from X.739.

## Imported Information Object Classes

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes

### Q.822

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822

#### Classes

##### «Q.822» CurrentData class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::«Q.822» CurrentData

Parent class: «X.739» Scanner

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» CurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| scannerId |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| operationalState |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1..3 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| administrativeState |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### «Q.822» HistoryData class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::«Q.822» HistoryData

Parent class: «X.721» Top

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» HistoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| historyDataId |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| periodEndTime |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatameasurementlistpkg | «Q.822» historyDataMeasurementListPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatasuspectintervalflagpkg | «Q.822» historyDataSuspectIntervalFlagPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» objectdeletenotificationpkg | «Q.822» objectDeleteNotificationPkg | 0..1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### «Q.822» ThresholdData class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::«Q.822» ThresholdData

Parent class: «X.721» Top

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» ThresholdData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| thresholdDataId |  | 1 | The thresholdData object class is a class of managed support objects that contains the values of the threshold settings for the PM parameters. At least one of the counterThresholdListPkg or the gaugeThresholdListPkg must be instantiated. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### Associations

#### Imported Arche Types

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types

##### Classes

###### «Q.822» filterSuppressionPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» filterSuppressionPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» filterSuppressionPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| discriminatorConstruct |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» historyDataMeasurementListPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» historyDataMeasurementListPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» historyDataMeasurementListPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cTPId |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» historyDataSuspectIntervalFlagPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» historyDataSuspectIntervalFlagPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» historyDataSuspectIntervalFlagPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cTPId |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» historyRetentionPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» historyRetentionPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» historyRetentionPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| historyRetention |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» maxSuppressedIntervalsPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» maxSuppressedIntervalsPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» maxSuppressedIntervalsPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| maxSuppressedIntervals |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» measurementListPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» measurementListPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» measurementListPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| measurementList |  | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» numSuppressedIntervalsPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» numSuppressedIntervalsPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» numSuppressedIntervalsPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numSuppressedIntervals |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» objectDeleteNotificationPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» objectDeleteNotificationPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» objectDeleteNotificationPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cTPId |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» observedManagedObjectPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» observedManagedObjectPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» observedManagedObjectPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| observedObjectClass |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| observedObjectInstance |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» scheduledPMReportPkg  class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» scheduledPMReportPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» scheduledPMReportPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| scanAttributeIdList |  | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| numericAttributeIdArray |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| onceReportAttributeIdList |  | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» thresholdPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» thresholdPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «Q.822» thresholdPkg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| reportAllAttributes |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| suppressAdditionalThresholds |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| thresholdDataInstance |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

###### «Q.822» zeroSuppressionPkg class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::Q.822::Imported Arche Types::«Q.822» zeroSuppressionPkg

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

### X.721

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.721

#### Classes

##### «X.721» Top class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.721::«X.721» Top

Description: This is the top level of managed object class hierarchy and every other managed object class is a specialization of either this generic class (top) or a specialization of subclass of top. The parameter miscellaneousError is to be used when a processing failure has occurred and the error condition encountered does not match any of object's defined specific error types.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «X.721» Top

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### Imported Arche Types

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.721::Imported Arche Types

##### Classes

###### «X.721» administrativeStatePackage class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.721::Imported Arche Types::«X.721» administrativeStatePackage

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «X.721» administrativeStatePackage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| administrativeState |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

### X.739

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.739

#### Classes

##### «X.739» Scanner class

Qualified Name: G.8052_v2.07-model::Imported Information Object Classes::X.739::«X.739» Scanner

Parent class: «X.721» Top

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class «X.739» Scanner

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| scannerId |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| operationalState |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1..3 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| administrativeState |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

## Object Classes

Qualified Name: G.8052_v2.07-model::Object Classes

### Classes

#### AddressTable class

Qualified Name: G.8052_v2.07-model::Object Classes::AddressTable

Description: An instance of this object class manages an address table of the Ethernet connection function of an NE.

Description: Basic attributes: crossConnectionRefList, stpLearningState, ageing, configuredTuples, groupDefaultPorts, isLearningEnabled, learnedTuples

Description: Basic operations: flushConfig, flushLearned, modifyAddressTable

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AddressTable

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| crossConnectionRefList | ETH_CrossConnection | 1..* | This attribute identifies the list of ETH_CrossConnection instances (i.e. flow forwarding processes) that use this address table. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| ageing | Integer | 1 | This attribute contains the Ageing time period for entries configured from the Learning Process. The value defines units of seconds. The value of zero implies "Never" aged. | OpenModelAttribute - valueRange: 0, 10 – 1 000 000 - support: MANDATORY - condition: none |
| isLearningEnabled | Boolean | 1 | This attribute specifies whether the learned address information will be used to update the address table (true) or not (false). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| learnedTuples | AddressTuple | 0..* | This attribute contains the tuples (Address, {ports}) learned via the learning process of the Ethernet connection function. See also section 9.1.1/G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| configuredTuples | AddressTuple | 0..* | This attribute contains the tuples (Address, {port}) configured via the modifyAddressTable operation (G.8021:MI_FF_ETH_FF). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| groupDefaultPorts | Integer | 0..* | This attribute identifies the ports for forwarding frames with destination Multicast MAC address which doesn't have a tuple (Address, {port}) in the Address table. It models the G.8021:MI_FF_Group_Default input to the Address Table. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| stpLearningState | Tp_Pac | 0..* | This association identifies the ports which are in the learning state. That means all frames received on that port will be discarded by the learning process, and therefore not forwarded to the forwarding process; however the (Address, {ports}) tuple may be submitted to the Address Table process before the frame is dropped (depending on the value of MI_FF_Learning). This attribute models the G.8021:MI_FF_STP_Learning_State[i]. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### AggregationPort class

Qualified Name: G.8052_v2.07-model::Object Classes::AggregationPort

Description: This object class the common properties of the Aggregator Source and Aggregator Sink object classes.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AggregationPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### AggregationPortSink class

Qualified Name: G.8052_v2.07-model::Object Classes::AggregationPortSink

Description: An instance of this object class manages the aggregation port of the ETH Link Aggregation Sink function. 
																				This object class models the ETYn-Np/ETH-LAG-Na_A_Sk_MI_AggPort[1..Np]_ information.

Parent class: AggregationPort

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AggregationPortSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### AggregationPortSource class

Qualified Name: G.8052_v2.07-model::Object Classes::AggregationPortSource

Description: Actor Process related attributes: actorAdminState, actorOperKey, actorOperState,actorPort, actorPortPriority

Description: An instance of this object class manages the aggregation port of the ETH Link Aggregation Source function. 
																				This object class models the ETYn-Np/ETH-LAG-Na_A_So_MI_AggPort[1..Np]_ information.
																				Note: 802.1AX clause 6.3.2.1 has the following additional attributes: Port ID, ActorSystemPriority, ActorSystemID, ActorAdminKey, PartnerAdminSystemPriority, PartnerAdminSystemID, PartnerAdminKey, SelectedAggID, AttachedAggID, PartnerAdminPort, PartnerAdminPortPriority, PartnerAdminState, and AggregateOrIndividual. These attributes have been omitted because they are specific to the IEEE management philosophy or for simplification in transport equipment.

Description: Partner Process related attributes: partnerOperKey, partnerOperPort, partnerOperPortPriority, partnerOperState, partnerOperSystemId, partnerOperSystemPriority

Parent class: AggregationPort

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AggregationPortSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| actorAdminState | BitString | 1 | See IEEE 802.1AX: A string of 8 bits, corresponding to the administrative values of Actor_State (5.4.2) as transmitted by the Actor in LACPDUs. The first bit corresponds to bit 0 of Actor_State (LACP_Activity), the second bit corresponds to bit 1 (LACP_Timeout), the third bit corresponds to bit 2 (Aggregation), the fourth bit corresponds to bit 3 (Synchronization), the fifth bit corresponds to bit 4 (Collecting), the sixth bit corresponds to bit 5 (Distributing), the seventh bit corresponds to bit 6 (Defaulted), and the eighth bit corresponds to bit 7 (Expired). These values allow administrative control over the values of LACP_Activity, LACP_Timeout, and Aggregation. This attribute value is read-write. | OpenModelAttribute - valueRange: SIZE(1..8) - support: MANDATORY - condition: none |
| actorOperKey | Integer | 1 | See IEEE 802.1AX: The current operational value of the Key for the Aggregation Port. This is a 16-bit read-only value. The meaning of particular Key values is of local significance. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| partnerOperSystemPriority | Integer | 1 | See IEEE 802.1AX: A 2-octet read-only value indicating the operational value of priority associated with the Partner's System ID. The value of this attribute may contain the manually configured value carried in aAggPortPartnerAdminSystemPriority if there is no protocol Partner. | OpenModelAttribute - valueRange: Size(1..16) - support: MANDATORY - condition: none |
| partnerOperSystemId | MacAddress | 1 | See IEEE 802.1AX: A 6-octet read-only MACAddress value representing the current value of the Aggregation Port's protocol Partner's System ID. A value of zero indicates that there is no known protocol Partner. The value of this attribute may contain the manually configured value carried in aAggPortPartnerAdminSystemID if there is no protocol Partner. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| partnerOperKey | Integer | 1 | See IEEE 802.1AX: The current operational value of the Key for the protocol Partner. The value of this attribute may contain the manually configured value carried in aAggPortPartnerAdminKey if there is no protocol Partner. | OpenModelAttribute - valueRange: 16 bit - support: MANDATORY - condition: none |
| actorPort | Integer | 1 | See IEEE 802.1AX: The port number locally assigned to the Aggregation Port. The port number is communicated in LACPDUs as the Actor_Port. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| actorPortPriority | Integer | 1 | See IEEE 802.1AX: The priority value assigned to this Aggregation Port. | OpenModelAttribute - valueRange: 16 bit - support: MANDATORY - condition: none |
| partnerOperPort | Integer | 1 | See IEEE 802.1AX: The operational port number assigned to this Aggregation Port by the Aggregation Port's protocol Partner. The value of this attribute may contain the manually configured value carried in aAggPortPartnerAdminPort if there is no protocol Partner. | OpenModelAttribute - valueRange: 16 bit - support: MANDATORY - condition: none |
| partnerOperPortPriority | Integer | 1 | See IEEE 802.1AX: The priority value assigned to this Aggregation Port by the Partner. The value of this attribute may contain the manually configured value carried in aAggPortPartnerAdminPortPriority if there is no protocol Partner. | OpenModelAttribute - valueRange: 16 bit - support: MANDATORY - condition: none |
| actorOperState | BitString | 1 | See IEEE 802.1AX: A string of 8 bits, corresponding to the current operational values of Actor_State (5.4.2) as transmitted by the Actor in LACPDUs. The bit allocations are as defined in 802.1AX clause 6.3.2.1.20. This attribute value is read-only. | OpenModelAttribute - valueRange: Size(1..8) - support: MANDATORY - condition: none |
| partnerOperState | BitString | 1 | See IEEE 802.1AX: A string of 8 bits, corresponding to the current values of Actor_State in the most recently received LACPDU transmitted by the protocol Partner. The bit allocations are as defined in 802.1AX clause 6.3.2.1.20. In the absence of an active protocol Partner, this value may reflect the manually configured value aAggPortPartnerAdminState. | OpenModelAttribute - valueRange: Size(1..8) - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### Aggregator class

Qualified Name: G.8052_v2.07-model::Object Classes::Aggregator

Description: This object class the common properties of the Aggregator Source and Aggregator Sink object classes.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Aggregator

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### AggregatorSink class

Qualified Name: G.8052_v2.07-model::Object Classes::AggregatorSink

Description: Basic attributes: filterConfig, isSsfReported, pllThr

Description: Defect correlation Process related attribute: currentProblemList

Description: An instance of this object class manages the aggregator of the ETH Link Aggregation Sink function.
																				This object class models the ETYn-Np/ETH-LAG-Na_A_Sk_MI_Agg[1..Na]_ information.

Parent class: Aggregator

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AggregatorSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pllThr | Integer | 1 | This attribute provisions the threshold for the number of active ports. If the number of active ports is more than zero but less than the provisioned threshold, a cPLL (Partial Link Loss) is raised. See section 9.7.1.2 of G.8021. | OpenModelAttribute - valueRange: 0..number of ports - support: MANDATORY - condition: none |
| isSsfReported | Boolean | 1 | This attribute provisions whether the SSF defect should be reported as fault cause or not. It models the ETH-LAG_FT_Sk_MI_SSF_Reported defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| filterConfig | ControlFrameFilter | 1 | This attribute models the FilterConfig MI defined in section 8.3/G.8021. It indicates the configured filter action for each of the 33 group MAC addresses for control frames. The 33 MAC addresses are: - All bridges address: 01-80-C2-00-00-10, - Reserved addresses: 01-80-C2-00-00-00 to 01-80-C2-00-00-0F, - GARP Application addresses: 01-80-C2-00-00-20 to 01-80-C2-00-00-2F. The filter action is Pass or Block. If the destination address of the incoming ETH_CI_D matches one of the above addresses, the filter process shall perform the corresponding configured filter action. If none of the above addresses match, the ETH_CI_D is passed. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | AggregatorProblemList | 0..* | This attribute identifies the active probable causes (failure) of the object. | OpenModelAttribute - valueRange: SSF, PLL, TLL - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### AggregatorSource class

Qualified Name: G.8052_v2.07-model::Object Classes::AggregatorSource

Description: Actor related attributes: actorOperKey, actorSystemId, actorSystemPriority

Description: Partner related attributes: partnerOperKey, partnerSystemId, partnerSystemPriority

Description: Basic attributes: collectorMaxDelay, dataRate

Description: An instance of this object class manages the aggregator of the ETH Link Aggregation Source function. 
																				This object class models the ETYn-Np/ETH-LAG-Na_A_So_MI_Agg[1..Na]_ information.
																				Note: 802.1AX clause 6.3.1.1 has the following additional attributes: AdminKey, MACAddress, AdminState (up/down), OperState (up/down), TimeOfLastOperChange. These attributes have been omitted because they are specific to the IEEE management philosophy or for simplification in transport equipment.

Parent class: Aggregator

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class AggregatorSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| actorSystemId | MacAddress | 1 | See 802.1AX: A MAC address used as a unique identifier for the System that contains this Aggregator. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| actorSystemPriority | Integer | 1 | See 802.1AX: Indicating the priority associated with the Actor's System ID. | OpenModelAttribute - valueRange: 2-octet - support: MANDATORY - condition: none |
| actorOperKey | Integer | 1 | See 802.1AX: The current operational value of the Key for the Aggregator. The administrative Key value may differ from the operational Key value for the reasons discussed in 5.6.2. The meaning of particular Key values is of local significance. | OpenModelAttribute - valueRange: 16 bit - support: MANDATORY - condition: none |
| partnerSystemId | MacAddress | 1 | See 802.1AX: A MAC address consisting of the unique identifier for the current protocol Partner of this Aggregator. A value of zero indicates that there is no known Partner. If the aggregation is manually configured, this System ID value will be a value assigned by the local System. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| partnerSystemPriority | Integer | 1 | See 802.1AX: Indicates the priority associated with the Partner's System ID. If the aggregation is manually configured, this System Priority value will be a value assigned by the local System. | OpenModelAttribute - valueRange: 2-octet - support: MANDATORY - condition: none |
| partnerOperKey | Integer | 1 | See 802.1AX: The current operational value of the Key for the Aggregator's current protocol Partner. If the aggregation is manually configured, this Key value will be a value assigned by the local System. | OpenModelAttribute - valueRange: 16-bit - support: MANDATORY - condition: none |
| dataRate | Integer | 1 | See 802.1AX: The current data rate, in bits per second, of the aggregate link. The value is calculated as N times the data rate of a single link in the aggregation, where N is the number of active links. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| collectorMaxDelay | Integer | 1 | See 802.1AX: The value of this attribute defines the maximum delay, in tens of microseconds, that may be imposed by the Frame Collector between receiving a frame from an Aggregator Parser, and either delivering the frame to its MAC Client or discarding the frame (see IEEE 802.1AX clause 5.2.3.1.1). | OpenModelAttribute - valueRange: 16-bit - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### CurrentData class

Qualified Name: G.8052_v2.07-model::Object Classes::CurrentData

Description: Basic operation: resetAllPmCounters

Description: Basic attribute: _associatedThresholdProfileList

Description: This object class is used to model the Ethernet specific Current Data.

Abstract class

Parent class: «Q.822» CurrentData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class CurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _associatedThresholdProfileList | ThresholdProfile | 0..* |  | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| scannerId |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| operationalState |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1..3 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| administrativeState |  | 1 |  | Inherited from Class «X.739» Scanner OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ETH_ConnectionFunction class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionFunction

Description: This object class models the Ethernet connection function of the NE defined in 9.1/G.8021. One and only one instance of this object class will be automatically instantiated.

Description: Basic operations: createCrossConnection, deleteCrossConnection, modifyCrossConnection

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionFunction

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _ethCrossConnection | ETH_CrossConnection | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _addressTable | AddressTable | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_ConnectionTerminationPoint class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionTerminationPoint

Description: This object class models the common properties of the ETH_ConnectionTerminationPointSink and ETH_ConnectionTerminationPointSource object classes.
																				It is a subclass of the MEP Control and MIP Control object classes to inherit the control operations for MEP and MIP (e.g. create/modify/delete/get MEP and MIP.

Description: Traffic shaping Process related operations: createTrafficShaping, modifyTrafficShaping, deleteTrafficShaping

Description: Auxiliary function related attribute: auxiliaryFunctionPositionSequence

Description: Traffic conditioning Process related operations: createTrafficConditioning, modifyTrafficConditioning, deleteTrafficConditioning

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionTerminationPoint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| auxiliaryFunctionPositionSequence | AuxiliaryFunctions | 0..* | This attribute indicates the positions (i.e., the relative order) of all the MEP, MIP, and TCS objects which are associated with the CTP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_ConnectionTerminationPointBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionTerminationPointBidirectional

Description: This object class inherits the properties of the ETH_ConnectionTerminationPointSink and ETH_ConnectionTerminationPointSource object classes.

Parent class: MipControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionTerminationPointBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _aggregator | Aggregator | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethTrafficConditioning | ETH_TrafficConditioning | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethTrafficShaping | ETH_TrafficShaping | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _mep | Mep | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _mipBidrectional | MipBidirectional | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _RapsCapableHalfMip | RapsCapableHalfMipBidirectional | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lagManager | LagManager | 1 | This attribute identifies the LAG Manager object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _aggregatorSource | AggregatorSource | 0..1 | This attribute identifies the aggregatorSource object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfConfig | CsfConfigSource_Pac | 0..1 | This association identifies the CsfConfigSource_Pac object which is only applicable under certain conditions defined in the constraint property of the association. | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _vlanConfig | VlanConfigSource_Pac | 0..1 | This association identifies the VlanConfigSource_Pac object that specifies the vlanConfig attribute. | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _csfRdiFdiEnable | CsfRdiFdiEnableSource_Pac | 0..1 |  | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _MepSource | MepSource | 0..* | This field contains the mepsource associationEnd documentation | Inherited from Class ETH_ConnectionTerminationPointSource OpenModelAttribute - valueRange: mepsource valueRange - support: MANDATORY - condition: none |
| auxiliaryFunctionPositionSequence | AuxiliaryFunctions | 0..* | This attribute indicates the positions (i.e., the relative order) of all the MEP, MIP, and TCS objects which are associated with the CTP. | Inherited from Class ETH_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _lagManager | LagManager | 1 | This association identifies the LAG Manager object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregatorSink | AggregatorSink | 0..1 | This association identifies the aggregatorSink object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfReport | CsfReportSink_Pac | 0..1 | This association identifies the CsfReportSink _Pac object which is only applicable under certain conditions defined in the constraint property of the association. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfRdiFdiEnable | CsfRdiFdiEnableSink_Pac | 0..1 | This association identifies the CsfRdiFdiEnableSink _Pac object which is only applicable under certain conditions defined in the constraint property of the association. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _vlanConfig | VlanConfigSink_Pac | 0..1 | This association identifies the VlanConfigSink_Pac objectt which is only applicable under certain conditions defined in the constraint property of the association. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _filterConfig | FilterConfigSink_Pac | 0..1 | This association identifies the FilterConfigSink_Pac objectt which is only applicable under certain conditions defined in the constraint property of the association. | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _mepSink | MepSink | 0..* |  | Inherited from Class ETH_ConnectionTerminationPointSink OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| auxiliaryFunctionPositionSequence | AuxiliaryFunctions | 0..* | This attribute indicates the positions (i.e., the relative order) of all the MEP, MIP, and TCS objects which are associated with the CTP. | Inherited from Class ETH_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### ETH_ConnectionTerminationPointPool class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionTerminationPointPool

Description: Basic operations: addCtps, removeCtps

Description: Basic attributes: _containedCtpRefList

Description: This object class models the grouping of a set of ETH_ConnectionTerminationPoint object instances (sink, source or bidirectional).

Parent class: G8052GlobalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionTerminationPointPool

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _containedCtpRefList | ETH_ConnectionTerminationPoint | 0..* | This attribute identifies the set of ETH_ConnectionTerminationPoinkSource instances, which are being multiplexed at the ETH/ETHG adaptation function. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_ConnectionTerminationPointSink class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionTerminationPointSink

Description: This object class models the client layer related sink function of all adaptation functions defined in G.8021 who has Ethernet as their client.

Description: VID DEMUX Process related attribute: _vlanConfig

Description: Defect correlations Process related attribute: _csfReport

Description: Consequent Actions Process related attribute: _csfRdiFdiEnable

Description: LAG Process related attributes: _lagManager, _aggregatorSink

Description: Filter Process related attribute: _filterConfig

Parent class: ETH_ConnectionTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionTerminationPointSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _lagManager | LagManager | 1 | This association identifies the LAG Manager object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregatorSink | AggregatorSink | 0..1 | This association identifies the aggregatorSink object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfReport | CsfReportSink_Pac | 0..1 | This association identifies the CsfReportSink _Pac object which is only applicable under certain conditions defined in the constraint property of the association. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfRdiFdiEnable | CsfRdiFdiEnableSink_Pac | 0..1 | This association identifies the CsfRdiFdiEnableSink _Pac object which is only applicable under certain conditions defined in the constraint property of the association. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _vlanConfig | VlanConfigSink_Pac | 0..1 | This association identifies the VlanConfigSink_Pac objectt which is only applicable under certain conditions defined in the constraint property of the association. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _filterConfig | FilterConfigSink_Pac | 0..1 | This association identifies the FilterConfigSink_Pac objectt which is only applicable under certain conditions defined in the constraint property of the association. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _mepSink | MepSink | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| auxiliaryFunctionPositionSequence | AuxiliaryFunctions | 0..* | This attribute indicates the positions (i.e., the relative order) of all the MEP, MIP, and TCS objects which are associated with the CTP. | Inherited from Class ETH_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_ConnectionTerminationPointSource class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConnectionTerminationPointSource

Description: VID MUX Process related attrbute: _vlanConfig

Description: Consequent actions Process related attributes: _csfConfig, _csfRdiFdiEnable

Description: This object class models the client layer related source function of all adaptation functions defined in G.8021 who has Ethernet as their client.

Description: LAG Process related attributes: _lagManager, _aggregatorSource

Parent class: ETH_ConnectionTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConnectionTerminationPointSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _lagManager | LagManager | 1 | This attribute identifies the LAG Manager object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _aggregatorSource | AggregatorSource | 0..1 | This attribute identifies the aggregatorSource object instance to which the ETH CTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _csfConfig | CsfConfigSource_Pac | 0..1 | This association identifies the CsfConfigSource_Pac object which is only applicable under certain conditions defined in the constraint property of the association. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _vlanConfig | VlanConfigSource_Pac | 0..1 | This association identifies the VlanConfigSource_Pac object that specifies the vlanConfig attribute. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _csfRdiFdiEnable | CsfRdiFdiEnableSource_Pac | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _MepSource | MepSource | 0..* | This field contains the mepsource associationEnd documentation | OpenModelAttribute - valueRange: mepsource valueRange - support: MANDATORY - condition: none |
| auxiliaryFunctionPositionSequence | AuxiliaryFunctions | 0..* | This attribute indicates the positions (i.e., the relative order) of all the MEP, MIP, and TCS objects which are associated with the CTP. | Inherited from Class ETH_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_ConstraintDomain class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_ConstraintDomain

Description: The object class models the network element that supports the transport Ethernet functions. An instance of this object class or sub-class may be a hybrid network element that contains additional functions of other transport technologies.

Description: Threshol profile related operations: createThresholdProfile, deleteThresholdProfile, gettAllContainedThresholdProfileNames, getThresholdProfile, modifyThresholdProfile

Parent class: G8052NetworkElement

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_ConstraintDomain

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _lagmanager | LagManager | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethCtpPool | ETH_ConnectionTerminationPointPool | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethConnectionFunction | ETH_ConnectionFunction | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _etyTtpBidirectional | ETHnull_TrailTerminationPointBidirectional | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethTtpBidirectional | ETH_TrailTerminationPointBidirectional | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _physicalSubsystem | PhysicalSubsystem | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ThresholdProfile | ThresholdProfile | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _ethTtpSource | ETH_TrailTerminationPointSource | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _ethTtpSink | ETH_TrailTerminationPointSink | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _fdRefList |  | 0..* | Represents the FD that is completely within the boundary of the NE. At a low level of recursion, an FD (within a network element (NE)) represents a switch matrix (i.e., a fabric). Note that an NE can encompass multiple switch matrices (FDs) and the FD representing the switch matrix can be further partitioned. Where an FD is referenced by the NeEncompassesFd association, any FDs that it encompasses (i.e., that are associated with it by HigherLevelFdEncompassesLowerLevelFds), must also be encompassed by the NE and hence must have the NeEncompassesFd association. | Inherited from Class G8052NetworkElement OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ltppList |  | 0..* | OBSOLETE. Was reference to LtpPool. The pool has now been subsumed into the LTP. This will be deleted in the next release. | Inherited from Class G8052NetworkElement OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ltpRefList |  | 0..* | An NE has associated LTPs that are at its boundary. The NeEncompassesFd association occurs for FDs that are within the bounds of the NetworkElement definition such that the FD is bounded by LTPs, all of which are on the boundary of the NetworkElement or are within the NetworkElement. An LTP can be independent of an NE. | Inherited from Class G8052NetworkElement OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_CrossConnection class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_CrossConnection

Description: Port group related operations: createPortGroups, deletePortGroups, modifyPortGrops

Description: This object class models the Ethernet flow forwarding function, as defined in 9.1/G.8021. Each ETH_C flow forwarding process is represented by an instance of this object class.

Description: Basic attributes: _addressTableRef, _terminationPointList, connectionType, groupDefaultMode

Description: Ring automatic Protection Switching related attribute: _ethRapsGroup

Description: SNC Protection related attribute: _ethSncpGroup

Description: Port group Process related attribute: _portGroupList

Description: Basic operaton: MI_FF_Set_PortIds

Parent class: G8052GlobalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_CrossConnection

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _addressTableRef | AddressTable | 1 | This attribute identifies the Address Table instance that is used by the cross connection for forwarding. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| connectionType | ConnectionType | 1 | This attribute contains the type of the cross connection. TBD in G.8021. Allowed values: - twoPort-point-to-point (unidirectional, no learning) - twoPort-point-to-point (bidirectional, no learning) - nPort(n>2)-point-to-multipoint (unidirectional, no learning) - nPort(n>2)-rooted-multipoint (bidirectional, learning) - nPort(n>2)-multipoint -to-multipoint (bidirectional, learning) | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _portGroupList | ETH_PortGroup | 0..* | This attribute models the port group feature of the flow forwarding function. Ports belong to the same port group have restriction regarding forwarding. Frames from a port are not forwarded to the ports in the same port group. Note that the Port Group feature is current described in Appendix VII as asymmetric VLAN of G.8021. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| groupDefaultMode | GroupDefaultMode | 1 | This attribute defines the default group filtering behavior as specified in IEEE 802.1 section 8.8.6. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _terminationPointList | Tp_Pac | 2..* | This attribute identifies the terminnation points (i.e. ports) that have connectivity among them, except for the restriction as configured in the portGroupList attribute. Connectivity among termination points is configured via the create/modify/delete Cross Connection operations defined in the ETH_ConnectionFunction object. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _ethSncpGroup | ETH_SubNetworkConnectionProtectionGroup | 0..1 | This attribute models the SNC Protection feature of the cross connection. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _eth_ringautomaticprotectionswitchinggroup | ETH_RingAutomaticProtectionSwitchingGroup | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_PortGroup class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_PortGroup

Description: Basic attribute: _terminationPointList

Description: See G.8021 section 9.1.1
																				This object class models the port group function of G.8021 (Appendix VII). An instance of this object class consists of a group of ports such that direct traffic communication among members of the port group is disabled.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_PortGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _terminationPointList | Tp_Pac | 2..* | This association identifies a set of termination points (ports) so that direct traffic communication among members of the list is disabled. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_RingAutomaticProtectionSwitchingGroup class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_RingAutomaticProtectionSwitchingGroup

Description: Basic operations: clearSwitch, forceSwitch, manualSwitch

Description: Basic attributes: _associatedMepRefList, compatibleVersion, controlProcessType, guardTime, holdOffTime, mustProgagateTopologyChange, ringIdentifier, ringPortRefList

Description: This object class represents the participation of the Network Element in Ring Protections. If the NE is involved in many Ring Protection schemes, there is one ETH_RAPS_Group object instance per Ring Protection scheme.
																				See G.8021 and G.8032.
																				Note: The object class inherits the protectionSwitchNotification from M.3160:ProtectionGroup.
																				Additional information on inherited attributes:
																				- revertive: default value = true; readOnly = false; AVC = YES.
																				- waitToRestoreTime: defaultValue = 300 seconds; readOnly = false; AVC = YES; nullable = true; valueRange = 60..720 seconds in steps of 60 seconds; only applicable if rplOwnerNode is not empty.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_RingAutomaticProtectionSwitchingGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mustPropagateTopologyChange | Boolean | 1 | See clause 10.1.12 of G.8032. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| compatibleVersion | String | 1 | This attribute represents the backward compatibility logic. See clause 10.1.13 of G.8032. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| controlProcessType | ControlProcessType | 1 | This attribute models the type of the R-APS Group (node) in the NE defined in section 10.1.14 and section 9.7 Figure 9-8 of G.8032 (02/2012). An example showing the usage of the allowed values is provided in the ControlProcessType enumeration definition. Note: Not all values of the attribute can be changed directly by the general set operation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| guardTime | Integer | 1 | This attribute defines the guard time. It can be configured in steps of 10 ms. Note that this attribute is related to the "wait to block time" which is equal to the guard time + 5 seconds. See G.8032. | OpenModelAttribute - valueRange: 10..2000 ms - support: MANDATORY - condition: none |
| holdOffTime | Integer | 1 | This attribute defines the hold off time. It can be configured in steps of 100 ms. See clause 10.1.8 of G.8032. | OpenModelAttribute - valueRange: 0..10.000 ms - support: MANDATORY - condition: none |
| ringIdentifier | Integer | 1 | This attribute indicates the ring ID of an ethernet ring protection instance. See section 10.1.1 in G.8032. | OpenModelAttribute - valueRange: 1..239 - support: MANDATORY - condition: none |
| _associatedMepRefList | MepBidirectional | 1..2 | This association identifies the MEPs which will provide the APS signal. In case of an edge node participating in a sub-ring without a virtual channel, only one MEP is associated. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ringPortRefList | RingPortRef | 1..2 | This attribute identifies the ETH CTPs that contain the RAPS-capable half MIPs which carries the RAPS signal. The syntax of this attribute identifies one or two ETH CTPs and their role in the RPL (i.e. either owner port, neighbour port, or normal port) with the constraint that in case of two CTP instances in the attribute, at least one of them has to be a normal port. See G.8032 for further details. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_SubNetworkConnectionProtectionGroup class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_SubNetworkConnectionProtectionGroup

Description: This object class represents the "Subnetwork Connection Protection Process" defined in G.8021. It supports only 1+1 and 1:1 protection schemes as defined in G.8031.
																				Note: The object class inherits the protectionSwitchNotification from M.3160:ProtectionGroup.
																				Additional information on inherited attributes:
																				- revertive: default value = true; readOnly = false; AVC = YES.
																				- protectionGroupType: not used
																				- waitToRestoreTime: defaultValue = 300 seconds; readOnly = false; AVC = YES; nullable = false; valueRange = 300..720 seconds in steps of 60 seconds

Description: Basic attributes: _associatedMepRef, _protectedFp, _protectingFp, _workingFp, holdOffTime, isSdProtectionEnabled, protectionBridgeType, sncpGroupState

Description: Basic operations: clearExternalCommandAndWaitToRestoreState, clearLocalFreeze, exercise, forceSwitch, localFreeze, lockoutProtection, manualSwitch

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_SubNetworkConnectionProtectionGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _workingFp | Tp_Pac | 1 | This association identifies the working port (ETH_FP). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _protectingFp | Tp_Pac | 1 | This association identifies the protecting port (ETH_FP). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| protectionBridgeType | ProtectionBridgeType | 1 | This attribute models the ETH_C_MI_PS_ProtType and the ETH_C_MI_PS_BridgeType management information defined in G.8021. See section 11.4 of G.8031. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| holdOffTime | Integer | 1 | This attribute models the hold off timer of the protection scheme. The value can be defined in steps of 100 ms. See section 11.12 in G.8031. | OpenModelAttribute - valueRange: 0 .. 10.000 ms - support: MANDATORY - condition: none |
| sncpGroupState | SncpGroupState | 1 | This attribute indicates the state of the protection group. See G.8031 Appendix IV. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _protectedFp | Tp_Pac | 0..1 | This association identifies the port (EH_FP) that is protected by the protection group. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isSdProtectionEnabled | Boolean | 1 | This attribute indicates whether signal degrade (SD) protection is enabled or not. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _associatedMepRef | MepBidirectional | 1 | This association identifies the MEPs which will provide the APS signal. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### ETH_TrafficConditioning class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrafficConditioning

Description: This object class models the ETH traffic conditioning function as defined in G.8021.

Description: Basic attributes: codirectional, condConfigList, prioConfigList

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrafficConditioning

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| prioConfigList | PriorityConfiguration | 1..8 | This attribute indicates the Priority Splitter function for the mapping of the Ethernet frame priority (ETH_CI_P) values to the output queue. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| condConfigList | TrafficConditioningConfiguration | 1..8 | This attribute indicates for the conditioner process the conditioning parameters: - Queue ID: Indicates the Queue ID - Committed Information Rate (CIR): number of bits per second - Committed Burst Size (CBS): number of bytes - Excess Information Rate (EIR): number of bits per second - Excess Burst Size (EBS): number of bytes - Coupling flag (CF): 0 or 1 - Color mode (CM): color-blind and color-aware. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| codirectional | Boolean | 1 | This attribute indicates the direction of the conditioner. The value of true means that the conditioner (modeled as a TCS Sink according to G.8021) is associated with the sink part of the containing CTP. The value of false means that the conditioner (modeled as a TCS Sink according to G.8021) is associated with the source part of the containing CTP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_TrafficShaping class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrafficShaping

Description: This object class models the ETH traffic shaping function as defined in G.8021.

Description: Basic attribute: codirectional, prioConfigList, queueConfigList, schedConfig

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrafficShaping

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| prioConfigList | PriorityConfiguration | 1..8 | This attribute configures the Priority Splitter function for the mapping of the Ethernet frame priority (ETH_CI_P) values to the output queue. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| queueConfigList | QueueConfiguration | 1..8 | This attribute configures the Queue depth and Dropping threshold parameters of the Queue process. The Queue depth sets the maximum size of the queue in bytes. An incoming ETH_CI traffic unit is dropped if there is insufficient space in the queue to hold the whole unit. The Dropping threshold sets the threshold of the queue. If the queue is filled beyond this threshold, incoming ETH_CI traffic units accompanied by the ETH_CI_DE signal set are dropped. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| schedConfig | SchedulingConfiguration | 1 | This attribute configures the scheduler process. The value of this attribute is for further study because it is for further study in G.8021. Scheduler is a pointer to a Scheduler object, which is to be defined in the future (because in G.8021, this is FFS). Note that the only significance of the GTCS function defined in G.8021 is the use of a common scheduler for shaping. Given that, G.8052 models the common scheduler feature by having a common value for this attribute. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| codirectional | Boolean | 1 | This attribute indicates the direction of the shaping function. The value of true means that the shaping (modeled as a TCS Source according to G.8021) is associated with the source part of the containing CTP. The value of false means that the shaping (modeled as a TCS Source according to G.8021) is associated with the sink part of the containing CTP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrailTerminationPoint

Description: This object class models the Ethernet Flow Termination function located at a layer boundary.

Abstract class

Parent class: G8052GlobalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrailTerminationPoint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_TrailTerminationPointBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrailTerminationPointBidirectional

Description: This object class models the Ethernet Flow Termination bidirectional function located at a layer boundary.

Parent class: ETH_TrailTerminationPointSource

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrailTerminationPointBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _ethCtpBidirectional | ETH_ConnectionTerminationPointBidirectional | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _mccCtp | MCC_ConnectionTerminationPoint | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _mep | Mep | 1..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _multiplexing | EthServerTtpSource_Pac | 0..1 | This navigable attribute models the sever layer (Ethernet) related information of the ETH/ETH-m source function. This information is modeled as a conditional set (Pac) of attributes specified in EthServerTtpSource_Pac. | Inherited from Class ETH_TrailTerminationPointSource OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _ethCtpSource | ETH_ConnectionTerminationPointSource | 0..* |  | Inherited from Class ETH_TrailTerminationPointSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _MepSource | MepSource | 1..* |  | Inherited from Class ETH_TrailTerminationPointSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _multiplexing | EthServerTtpSink_Pac | 0..1 | This navigable attribute models the sever layer (Ethernet) related information of the ETH/ETH-m source function. This information is modeled as a conditional set (Pac) of attributes specified in EthServerTtpSink_Pac. | Inherited from Class ETH_TrailTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | ETH_TtpProblemList | 0..* | This attribute indicates the currently active failure(s) of the entity. | Inherited from Class ETH_TrailTerminationPointSink OpenModelAttribute - valueRange: SSF, LCK, LOC[i], MMG, UNM, UNP, UNPri, UNL, DEG, RDI - support: MANDATORY - condition: none |
| _ethCtpSink | ETH_ConnectionTerminationPointSink | 0..* |  | Inherited from Class ETH_TrailTerminationPointSink OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _MepSink | MepSink | 1..* |  | Inherited from Class ETH_TrailTerminationPointSink OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ETH_TrailTerminationPointSink class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrailTerminationPointSink

Description: This object class models the Ethernet Flow Termination Sink function (ETH_FT_Sk) located at a layer boundary.

Description: Defect correlation Process related attribute: currentProblemList

Description: ETH multiplexing server sink related attribute: _multiplexing, which is conditional package consisting of attributes: adminState, etherType, priorityCodePointConfig, priorityRegenerate, portVid, frametypeConfig, filterConfig

Parent class: ETH_TrailTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrailTerminationPointSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _multiplexing | EthServerTtpSink_Pac | 0..1 | This navigable attribute models the sever layer (Ethernet) related information of the ETH/ETH-m source function. This information is modeled as a conditional set (Pac) of attributes specified in EthServerTtpSink_Pac. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | ETH_TtpProblemList | 0..* | This attribute indicates the currently active failure(s) of the entity. | OpenModelAttribute - valueRange: SSF, LCK, LOC[i], MMG, UNM, UNP, UNPri, UNL, DEG, RDI - support: MANDATORY - condition: none |
| _ethCtpSink | ETH_ConnectionTerminationPointSink | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _MepSink | MepSink | 1..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETH_TrailTerminationPointSource class

Qualified Name: G.8052_v2.07-model::Object Classes::ETH_TrailTerminationPointSource

Description: ETH multiplexing server source related attribute: _multiplexing, which is a conditional package consisting of attributes: adminState, etherType, priorityCodePointConfig

Description: This object class models the Ethernet Flow Termination Source function (ETH_FT_So) located at a layer boundary.

Parent class: ETH_TrailTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETH_TrailTerminationPointSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _multiplexing | EthServerTtpSource_Pac | 0..1 | This navigable attribute models the sever layer (Ethernet) related information of the ETH/ETH-m source function. This information is modeled as a conditional set (Pac) of attributes specified in EthServerTtpSource_Pac. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _ethCtpSource | ETH_ConnectionTerminationPointSource | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _MepSource | MepSource | 1..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETHnull_TrailTerminationPointBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::ETHnull_TrailTerminationPointBidirectional

Description: This object class models the Ethernet PHY Layer (ETY) trail termination functions (ETYn_TT) defined in G.8021.

Parent class: ETHnull_TrailTerminationPointSink

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETHnull_TrailTerminationPointBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _aggregationPort | AggregationPort | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ethCtpBidirectional | ETH_ConnectionTerminationPointBidirectional | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _lagManager | LagManager | 1 | This navigable attribute identifies the LAG Manager object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | Inherited from Class ETHnull_TrailTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregationPortSink | AggregationPortSink | 0..1 | This navigable attribute identifies the aggregationPortSink object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | Inherited from Class ETHnull_TrailTerminationPointSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | ETY_TtpProblemList | 0..* | This attribute indicates the currently active failure(s) of the entity. | Inherited from Class ETHnull_TrailTerminationPointSink Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isFtsEnabled | Boolean | 1 | This attribute indicates whether Forced Transmitter Shutdown (FTS) is enabled or not. It models the ETYn_TT_So_MI_FTSEnable information. | Inherited from Class ETHnull_TrailTerminationPointSource Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| phyType | ETY_PhyType | 1 | This attribute identifies the PHY type of the ETY trail termination. See IEEE 802.3 clause 30.3.2.1.2. | Inherited from Class ETHnull_TrailTerminationPointSource Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| phyTypeList | ETY_PhyType | 0..* | This attribute identifies the possible PHY types that could be supported at the ETY trail termination. See IEEE 802.3 clause 30.3.2.1.3. | Inherited from Class ETHnull_TrailTerminationPointSource Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _lagManager | LagManager | 1 | This navigable attribute identifies the LAG Manager object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | Inherited from Class ETHnull_TrailTerminationPointSource OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isTxPauseEnabled | Boolean | 1 | This attribute identifies whether the Transmit Pause process is enabled or not. It models the MI_TxPauseEnable defined in G.8021. | Inherited from Class ETHnull_TrailTerminationPointSource Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregationPortSource | AggregationPortSource | 0..1 | This navigable attribute identifies the aggregationPortSource object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | Inherited from Class ETHnull_TrailTerminationPointSource OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### ETHnull_TrailTerminationPointSink class

Qualified Name: G.8052_v2.07-model::Object Classes::ETHnull_TrailTerminationPointSink

Description: Defect correlation Process related attribute: currentProblemList

Description: This object class models the ETYn_TT_Sk function and also the sever layer (i.e. ETY) related ETY/ETH adaptation function, which are defined in G.8021.

Description: LAG Process related attributes: _lagManager, _aggregationPortSink

Parent class: ETY_TrailTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETHnull_TrailTerminationPointSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _lagManager | LagManager | 1 | This navigable attribute identifies the LAG Manager object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregationPortSink | AggregationPortSink | 0..1 | This navigable attribute identifies the aggregationPortSink object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | ETY_TtpProblemList | 0..* | This attribute indicates the currently active failure(s) of the entity. | Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETHnull_TrailTerminationPointSource class

Qualified Name: G.8052_v2.07-model::Object Classes::ETHnull_TrailTerminationPointSource

Description: Transmit pause Process related attribute: isTxPauseEnabled

Description: Basic attributes: phyType, phyTypeList

Description: This object class models the ETYn_TT_So function and also the sever layer (i.e. ETY) related ETY/ETH adaptation function, which are defined in G.8021.

Description: Fault propagation Process related attribute: isFtsEnabled

Description: LAG Process related attributes: _lagManager, _aggregationPortSource

Parent class: ETY_TrailTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETHnull_TrailTerminationPointSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isFtsEnabled | Boolean | 1 | This attribute indicates whether Forced Transmitter Shutdown (FTS) is enabled or not. It models the ETYn_TT_So_MI_FTSEnable information. | Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| phyType | ETY_PhyType | 1 | This attribute identifies the PHY type of the ETY trail termination. See IEEE 802.3 clause 30.3.2.1.2. | Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| phyTypeList | ETY_PhyType | 0..* | This attribute identifies the possible PHY types that could be supported at the ETY trail termination. See IEEE 802.3 clause 30.3.2.1.3. | Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _lagManager | LagManager | 1 | This navigable attribute identifies the LAG Manager object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isTxPauseEnabled | Boolean | 1 | This attribute identifies whether the Transmit Pause process is enabled or not. It models the MI_TxPauseEnable defined in G.8021. | Deprecated OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _aggregationPortSource | AggregationPortSource | 0..1 | This navigable attribute identifies the aggregationPortSource object instance to which the ETY TTP is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ETY_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Object Classes::ETY_TrailTerminationPoint

Description: This object class models the common parts of the ETYn_TT function, which are defined in G.8021.

Abstract class

Parent class: G8052GlobalClass

Applied Stereotypes:

- Deprecated

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ETY_TrailTerminationPoint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | Inherited from Class G8052GlobalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| alarmStatus |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | Inherited from Class Tp_Pac OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | Inherited from Class G8052LayerProtocol OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### G8052GlobalClass class

Qualified Name: G.8052_v2.07-model::Object Classes::G8052GlobalClass

Description: Represents a type of thing (an Entity) that has instances which can exist in their own right (independently of any others).
																				Entity: Has identity, defined boundary, properties, functionality and lifecycle in a global context.
																				(consider in the context of an Object Class: (usage) The representation of a thing that may be an entity or an inseparable Entity Feature)

Abstract class

Parent class:

Applied Stereotypes:

- OpenModelClass

  - support: 
       MANDATORY

Table 1 Attributes for Class G8052GlobalClass

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localIdList | NameAndValue | 0..* | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| uuid | String | 1 | UUID: An identifier that is universally unique (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself globally unique, and immutable. An identifier carries no semantics with respect to the purpose or state of the entity) | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### G8052LayerProtocol class

Qualified Name: G.8052_v2.07-model::Object Classes::G8052LayerProtocol

Description: Each transport layer is represented by a LayerProtocol (LP) instance. The LayerProtocol instances it can be used for controlling termination and monitoring functionality. 
																				It can also be used for controlling the adaptation (i.e. encapsulation and/or multiplexing of client signal), tandem connection monitoring, traffic conditioning and/or shaping functionality at an intermediate point along a connection. 
																				Where the client – server relationship is fixed 1:1 and immutable, the layers can be encapsulated in a single LTP instance. Where the is a n:1 relationship between client and server, the layers must be split over two separate instances of LTP.

Abstract class

Parent class:

Applied Stereotypes:

- OpenModelClass

  - support: 
       MANDATORY

Table 1 Attributes for Class G8052LayerProtocol

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| layerProtocolName |  | 1 | Indicate the specific layer-protocol described by the LayerProtocol entity. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _lpSpec |  | 1 | The LpSpec identifies the interna structure of the LP explaining internal flexibilities, degree of termination and degree of adaptation on both client and server side. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| configuredClientCapacity |  | 0..1 | Provides a summarized view of the client capacity that is configurable for use. Note the cleint LTP association should provide all necessary detail hence this attribute is questionable. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| lpDirection |  | 1 | The overall directionality of the LP. - A BIDIRECTIONAL LP will have some SINK and/or SOURCE flowss. - A SINK LP can only contain elements with SINK flows or CONTRA_DIRECTION_SOURCE flows - A SOURCE LP can only contain SOURCE flows or CONTRA_DIRECTION_SINK flows | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| terminationState |  | 1 | Indicates whether the layer is terminated and if so how. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### G8052LocalClass class

Qualified Name: G.8052_v2.07-model::Object Classes::G8052LocalClass

Description: A LocalClass represents a Feature of an Entity. It is inseparable from a GlobalClass but is a distinct feature of that GlobalClass such that the instances of LocalClass are able to have associations to other instances..
																				Feature of an Entity: An inseparable, externally distinguishable part of an entity.
																				The mandatory LocalId of the LocalClass instance is unique in the context of the GlobalClass from which it is inseparable.
																				(consider in the context of an Object Class: (usage) The representation of a thing that may be an entity or an inseparable feature of an entity)

Abstract class

Parent class:

Applied Stereotypes:

- OpenModelClass

  - support: 
       MANDATORY

Table 1 Attributes for Class G8052LocalClass

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### G8052NetworkElement class

Qualified Name: G.8052_v2.07-model::Object Classes::G8052NetworkElement

Description: The Network Element (NE) object class represents a network element (traditional NE) in the data plane.
																				A data plane network element is essentially a consolidation of capabilities that can be viewed and controlled through a "single" management-control port.
																				In the direct interface from an SDN controller to a network element in the data plane, the NetworkElement object defines the scope of control for the resources within the network element
																				For example internal transfer of user information between the external terminations (ports of the NE), encapsulation, multiplexing/demultiplexing, and OAM functions, etc. 
																				The NetworkElement provides the scope of the naming space for identifying objects representing the resources within the data plane network element.
																				NE is really a product bundling or some view of management scope, management access, session. 
																				The NE is not directly part of topology but brings meaning to the FD context and the LTP context (and hence the links).

Abstract class

Parent class:

Applied Stereotypes:

- Deprecated

- OpenModelClass

  - support: 
       MANDATORY

Table 1 Attributes for Class G8052NetworkElement

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _fdRefList |  | 0..* | Represents the FD that is completely within the boundary of the NE. At a low level of recursion, an FD (within a network element (NE)) represents a switch matrix (i.e., a fabric). Note that an NE can encompass multiple switch matrices (FDs) and the FD representing the switch matrix can be further partitioned. Where an FD is referenced by the NeEncompassesFd association, any FDs that it encompasses (i.e., that are associated with it by HigherLevelFdEncompassesLowerLevelFds), must also be encompassed by the NE and hence must have the NeEncompassesFd association. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ltppList |  | 0..* | OBSOLETE. Was reference to LtpPool. The pool has now been subsumed into the LTP. This will be deleted in the next release. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _ltpRefList |  | 0..* | An NE has associated LTPs that are at its boundary. The NeEncompassesFd association occurs for FDs that are within the bounds of the NetworkElement definition such that the FD is bounded by LTPs, all of which are on the boundary of the NetworkElement or are within the NetworkElement. An LTP can be independent of an NE. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### LagManager class

Qualified Name: G.8052_v2.07-model::Object Classes::LagManager

Description: Aggregation Port related operations: addEtyTtps, removeEtyTtps

Description: Basic attributes: _ethCtpSinkRefList, _ethCtpSourceRefList, _etyTtpSinkRefList, _etyTtpSourceRefList, lagIdentifier

Description: This object class models the Ethernet link aggregation functions for adding and removing aggregators and/or aggregation ports to or from the aggregation.

Description: Aggregator related operations: addEthCtps, removeEthCtps

Description: LAG function control related attribute: isActive

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class LagManager

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| lagIdentifier | Integer | 1 | This attribute identifies the instance of the LagManager object. | OpenModelAttribute - valueRange: positive integer - support: MANDATORY - condition: none |
| _ethnullTtpSinkRefList | ETHnull_TrailTerminationPointSink | 0..1 | This navigable attribute identifies the ETY_TrailTerminationPointSink object instances to which the LagManager is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _ethnullTtpSourceRefList | ETHnull_TrailTerminationPointSource | 0..1 | This navigable attribute identifies the ETY_TrailTerminationPointSource object instances to which the LagManager is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _ethCtpSourceRefList | ETH_ConnectionTerminationPointSource | 0..1 | This navigable attribute identifies the ETH_ConnectionTerminationPointSource object instances to which the LagManager is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _ethCtpSinkRefList | ETH_ConnectionTerminationPointSink | 0..1 | This navigable attribute identifies the ETH_ConnectionTerminationPointSink object instances to which the LagManager is associated for Ethernet Link Aggregation. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isActive | Boolean | 1 | This attribute indicates whether the LagManager instance is active. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MCC_ConnectionTerminationPoint class

Qualified Name: G.8052_v2.07-model::Object Classes::MCC_ConnectionTerminationPoint

Description: This object class models the client specific (MCC) of the ETH/MCC adaptation function.

Description: Basic attributes: mel, mepMac

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MCC_ConnectionTerminationPoint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MCC_ConnectionTerminationPointBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::MCC_ConnectionTerminationPointBidirectional

Description: This object class models the bidirectional client specific (MCC) of the ETH/MCC adaptation function.

Parent class: MCC_ConnectionTerminationPointSink

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MCC_ConnectionTerminationPointBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| receivedExpectedDefectMessage | ExpectedDefectMessage | 1 | This attribute models the received expected defect message. | Inherited from Class MCC_ConnectionTerminationPointSink OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| mccPriority | Integer | 1 | This attribute is the priority of the expected defect message. | Inherited from Class MCC_ConnectionTerminationPointSource OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| mepIdentifier | Integer | 1 | This attribute identifies the peer MEP. | Inherited from Class MCC_ConnectionTerminationPointSource OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| expectedDefectMessageEnable | Boolean | 1 | This attribute enables the sending of expected defect messages. | Inherited from Class MCC_ConnectionTerminationPointSource OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| expectedDefectMessagePeriod | Integer | 1 | This attribute configures how often to send expected defect messages. | Inherited from Class MCC_ConnectionTerminationPointSource OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| expectedDefectMessageDuration | Integer | 1 | This attribute configures the duration for sending expected defect messages. | Inherited from Class MCC_ConnectionTerminationPointSource OpenModelAttribute - valueRange: Non negative (in seconds) - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MCC_ConnectionTerminationPointSink class

Qualified Name: G.8052_v2.07-model::Object Classes::MCC_ConnectionTerminationPointSink

Description: This object class models the sink side of the client specific (MCC) of the ETH/MCC adaptation function.

Description: Expected defect Process related attribute: receivedExpectedDefectMessage

Parent class: MCC_ConnectionTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MCC_ConnectionTerminationPointSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| receivedExpectedDefectMessage | ExpectedDefectMessage | 1 | This attribute models the received expected defect message. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MCC_ConnectionTerminationPointSource class

Qualified Name: G.8052_v2.07-model::Object Classes::MCC_ConnectionTerminationPointSource

Description: Expected defect Process related attributes: expectedDefectMessageDuration, expectedDefectMessageEnable, expectedDefectMessagePeriod

Description: Basic attributes: mccPriority, mepIdentifier

Description: This object class models the source side of the client specific (MCC) of the ETH/MCC adaptation function.

Parent class: MCC_ConnectionTerminationPoint

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MCC_ConnectionTerminationPointSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mccPriority | Integer | 1 | This attribute is the priority of the expected defect message. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| mepIdentifier | Integer | 1 | This attribute identifies the peer MEP. | OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| expectedDefectMessageEnable | Boolean | 1 | This attribute enables the sending of expected defect messages. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| expectedDefectMessagePeriod | Integer | 1 | This attribute configures how often to send expected defect messages. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| expectedDefectMessageDuration | Integer | 1 | This attribute configures the duration for sending expected defect messages. | OpenModelAttribute - valueRange: Non negative (in seconds) - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class MCC_ConnectionTerminationPoint OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MeasurementJobControl class

Qualified Name: G.8052_v2.07-model::Object Classes::MeasurementJobControl

Description: Proactive meausrement job related operations: disabeProActiveMeasurementJob, enableProActiveMeasurementJob, terminateProActiveMeasurementJob, getAllContainedMeasurementJobs, getCurrentDataValues, getHistoryDataValues

Description: This object class models the function of managing Measurement Jobs, such as enable/disable/terminate/get Measurement Jobs and retrieve PM data. The object class can be subclassed by MEP Source and MEP Sink for inheriting the management functions.

Description: On demand measurement job related operation: abortOnDemandMeasurementJob

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MeasurementJobControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### Mep class

Qualified Name: G.8052_v2.07-model::Object Classes::Mep

Description: Basic attributes: adminState, clientMel, megIdentifier, mepMac

Description: Continuity Check Process related attributes: ccPeriod, ccPriority, isCcEnabled

Description: This object class models the MEP functions that are common to MEP Sink and MEP Source.

Description: Lock Process related attributes: lckPeriod, lckPriority

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Mep

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values are: 3.33 ms for PS, 100 ms for PM, 1 s for FM. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MepBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::MepBidirectional

Description: SNCP related attribute: _associatedSncpGroupRef

Description: This object class models instance of MEP which supports both the MEP Sink and MEP Source functions.

Description: Proactive measurement job related operation: establishProActiveSingleEndedMeasurementJob

Description: On demand measurement job related operaton: establishOnDemandSingleEndedMeasurementJob

Description: RAPS related attribute: _associatedRapsGroupRef

Parent class: MepSink

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MepBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _associatedRapsGroupRef | ETH_RingAutomaticProtectionSwitchingGroup | 0..1 | This association identifies the associated RAPS Group to which the APS signal has to be forwarded. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _associatedSncpGroupRef | ETH_SubNetworkConnectionProtectionGroup | 0..1 | This association identifies the associated SNCP Group to which the APS signal has to be forwarded. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _onDemandSingleEndedMeasurementJobControl | OnDemandSingleEndedMeasurementJobControl | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _proactiveSingleEndedMeasurementJobControl | ProActiveSingleEndedMeasurementJobControl | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| peerMepRefList | Integer | 0..* | This attribute models the MI_PeerMEP_ID[i] signal defined in G.8021 and configured as specified in G.8051. It provides the identifiers of the MEPs which are peer to the subject MEP. | Inherited from Class MepSink OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| aisPeriod | OamPeriod | 1 | This attribute models the MI_AIS_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the AIS messages should be sent. | Inherited from Class MepSink OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| aisPriority | Integer | 1 | This attribute models the MI_AIS_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the AIS messages should be sent. | Inherited from Class MepSink OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCsfReported | Boolean | 1 | This attribute models the MI_CSF_Reported signal defined in G.8021 and configured as specified in G8051. It configures whether the secondary failure CSF should be reported or not. | Inherited from Class MepSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCsfRdiFdiEnabled | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable signal defined in G.8021 and configured as specified in G8051. | Inherited from Class MepSink OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | String | 1 | This attribute indicates the active alarms (i.e. current problems) of the MEP. | Inherited from Class MepSink OpenModelAttribute - valueRange: cLOC[i], cUNL, cMMG, cUNM, cDEG, cUNP, cUNPr, cRDI, cSSF, cLCK - support: MANDATORY - condition: none |
| bandwidthReport | BandwidthReport | 1 | This attribute models the content of the bandwidth report received by the MEP Sink from the peer MEP Source. | Inherited from Class MepSink OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| 1DmPriority | Integer | 1..* | This attribute indicates the list of 1DM priorities for the MepSink. | Inherited from Class MepSink OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _onDemandDualEndedMeasurementJobControlSink | OnDemandDualEndedMeasurementJobControlSink | 0..* |  | Inherited from Class MepSink OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _proactiveDualEndedMeasurementJobControlSink | ProActiveDualEndedMeasurementJobControlSink | 0..* |  | Inherited from Class MepSink OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values are: 3.33 ms for PS, 100 ms for PM, 1 s for FM. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| mepIdentifier | Integer | 1 | This attribute contains the identifier of the MEP. | Inherited from Class MepSource OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| csfConfig | CsfConfig | 1 | This attribute models the combination of all CSF related MI signals (MI_CSF_Enable, MI_CSFrdifdi_Enable, MI_CSFdci_Enable) as defined in G.8021. | Inherited from Class MepSource OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| csfPeriod | OamPeriod | 1 | This attribute models the MI_CSF_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CSF messages should be sent. | Inherited from Class MepSource OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| csfPriority | Integer | 1 | This attribute models the MI_CSF_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CSF messages should be sent | Inherited from Class MepSource OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| apsPriority | Integer | 1 | This attribute specifies the priority of the APS messages. See section 8.1.5 APS insert process in G.8021. | Inherited from Class MepSource OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| _proactiveDualEndedMeasurementJobControlSource | ProActiveDualEndedMeasurementJobControlSource | 0..* |  | Inherited from Class MepSource OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemandMeasurementJobControlSource | OnDemandDualEndedMeasurementJobControlSource | 0..* |  | Inherited from Class MepSource OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values are: 3.33 ms for PS, 100 ms for PM, 1 s for FM. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MepControl class

Qualified Name: G.8052_v2.07-model::Object Classes::MepControl

Description: This object class models the function of managing MEPs, such as create/modify/delete/get MEPs and retrieve PM data. The object class can be sub classed by ETH CTP and TTP for inheriting the MEP management functions.

Description: Basic operations: createMep, modifyMep, deleteMep, getAllContainedMeps

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MepControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MepSink class

Qualified Name: G.8052_v2.07-model::Object Classes::MepSink

Description: Defect correlation Process related attribute: currentProblemList

Description: Basic attribute: peerMepRefList

Description: AIS Process related attributes: aisPeriod, aisPriority

Description: Bandwidth notification Process related attribute: bandwidthReport

Description: 1DM related attribute: 1DmPriority

Description: This object class models the MEP sink function. Instance of this object class can be created and contained by ETH CTP or TTP objects.
																				It also provides the management of the dual-ended maintenance job, such as test.

Description: CSF Process related attributes: isCsfRdiFdiEnabled, isCsfReported

Abstract class

Parent class: MeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MepSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| peerMepRefList | Integer | 0..* | This attribute models the MI_PeerMEP_ID[i] signal defined in G.8021 and configured as specified in G.8051. It provides the identifiers of the MEPs which are peer to the subject MEP. | OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| aisPeriod | OamPeriod | 1 | This attribute models the MI_AIS_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the AIS messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| aisPriority | Integer | 1 | This attribute models the MI_AIS_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the AIS messages should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCsfReported | Boolean | 1 | This attribute models the MI_CSF_Reported signal defined in G.8021 and configured as specified in G8051. It configures whether the secondary failure CSF should be reported or not. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCsfRdiFdiEnabled | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList | String | 1 | This attribute indicates the active alarms (i.e. current problems) of the MEP. | OpenModelAttribute - valueRange: cLOC[i], cUNL, cMMG, cUNM, cDEG, cUNP, cUNPr, cRDI, cSSF, cLCK - support: MANDATORY - condition: none |
| bandwidthReport | BandwidthReport | 1 | This attribute models the content of the bandwidth report received by the MEP Sink from the peer MEP Source. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| 1DmPriority | Integer | 1..* | This attribute indicates the list of 1DM priorities for the MepSink. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _onDemandDualEndedMeasurementJobControlSink | OnDemandDualEndedMeasurementJobControlSink | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _proactiveDualEndedMeasurementJobControlSink | ProActiveDualEndedMeasurementJobControlSink | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values are: 3.33 ms for PS, 100 ms for PM, 1 s for FM. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MepSource class

Qualified Name: G.8052_v2.07-model::Object Classes::MepSource

Description: Basic attribute: mepIdentifier

Description: CSF Process related attributes: csfConfig, csfPeriod, csfPriority

Description: Test related operations: testInitiatorStart, testInitiatorTerminate

Description: On demand measurement job control related operation: establishOnDemandDualEndedMeasurementJobSource

Description: Loopback related operations: loopbackDiscover, loopbackSeries, loopbackTest, loopbackTestTerminate

Description: APS Process related attribute: apsPriority

Description: Proactive measurement job control related operation: establishProActiveDualEndedMeasurementJobSource

Description: This object class models the MEP source function. Instance of this object class can be created and contained by ETH CTP or TTP objects.
																				It also provides the management of single-ended maintenance jobs, such as loopback test, loopback discover, loopback series, link trace, and dual-ended maintenance job, such as test.

Description: Link trace related operation: linkTrace

Abstract class

Parent class: MeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MepSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepIdentifier | Integer | 1 | This attribute contains the identifier of the MEP. | OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| csfConfig | CsfConfig | 1 | This attribute models the combination of all CSF related MI signals (MI_CSF_Enable, MI_CSFrdifdi_Enable, MI_CSFdci_Enable) as defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| csfPeriod | OamPeriod | 1 | This attribute models the MI_CSF_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CSF messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| csfPriority | Integer | 1 | This attribute models the MI_CSF_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CSF messages should be sent | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| apsPriority | Integer | 1 | This attribute specifies the priority of the APS messages. See section 8.1.5 APS insert process in G.8021. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| _proactiveDualEndedMeasurementJobControlSource | ProActiveDualEndedMeasurementJobControlSource | 0..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemandMeasurementJobControlSource | OnDemandDualEndedMeasurementJobControlSource | 0..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values are: 3.33 ms for PS, 100 ms for PM, 1 s for FM. | Inherited from Class Mep OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Class Mep OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MipBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::MipBidirectional

Description: This object class models the MIP Sink and MIP Source functions. Only the bidirectional MIP is defined in the model.

Description: Basic attributes: isFullMip, mel, mipMac

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MipBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mipMac | MacAddress | 1 | This attribute contains the MAC address of the MIP instance. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MIP. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isFullMip | Boolean | 1 | This attribute indicates whether the MIP is a full MIP (true) or a down-half MIP (false). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### MipControl class

Qualified Name: G.8052_v2.07-model::Object Classes::MipControl

Description: Basic operations: createMip, deleteMip, getAllContainedMips

Description: This object class models the function of managing MIPs, such as create/delete/get MIPs.
																				It can be subclassed by ETH CTP Bidirectional for inheriting the MIP management functions.

Description: RAPS related opertion: createRapsCapableMip

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class MipControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### OnDemandDualEndedMeasurementJobControlSink class

Qualified Name: G.8052_v2.07-model::Object Classes::OnDemandDualEndedMeasurementJobControlSink

Description: This object class represents an on-demand measurement job controller sink for 1-way measurements. It is created as a result of an establishOnDemandDualEndedMeasurementJobSink() operation. It is deleted either automatically after the measurement job has completed (stop time reached) and the performance data AVC notification has been sent, or by an explicit abortOnDemandMeasurementJob() operation when the measurement job is running.

Description: Basic attributes: oamType, priority, sourceAddress, startTime, stopTime, testIdentifier

Description: Performance data related attribute: onDemandPerformanceData

Description: Performance data related operation: getIntermediateReport

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class OnDemandDualEndedMeasurementJobControlSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| startTime | GeneralizedTime | 1 | This attribute contains the start date and time of the on-demand session. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| stopTime | GeneralizedTime | 1 | This attribute contains the stop date and time of the on-demand session. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| sourceAddress | MacAddress | 1 | This attribute contains the MAC address of the peer MEP. See G.8013 for details. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| oamType | 1WayOamType | 1 | This attribute contains the type of the 1-way measurement. | OpenModelAttribute - valueRange: See corresponding Enum. - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. | OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| onDemandPerformanceData | OnDemandSinkPerformanceData | 1 | This attribute provides the final (at the end of the measurement interval) PM data report. The management system receives the final PM data report via the attributeValueChangeNotification of this attribute. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority of the OAM PDU frames. | OpenModelAttribute - valueRange: 0, 1, 2, 3, 4, 5, 6, 7 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### OnDemandDualEndedMeasurementJobControlSource class

Qualified Name: G.8052_v2.07-model::Object Classes::OnDemandDualEndedMeasurementJobControlSource

Description: Basic attribute: oamType

Description: This object class represents an on-demand measurement job controller source for 1-way measurements. It is created as a result of an establishOnDemandDualEndedMeasurementJobSource() operation. It is deleted either automatically after the measurement job has completed (stop time reached), or by an explicit abortOnDemandMeasurementJob() operation while the measurement job is running.

Parent class: OnDemandMeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class OnDemandDualEndedMeasurementJobControlSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| oamType | 1WayOamType | 1 | This attribute contains the type of the 1-way measurement. | OpenModelAttribute - valueRange: See corresponding Enum - support: MANDATORY - condition: none |
| startTime | GeneralizedTime | 1 | This attribute contains the start date and time of the on-demand session. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| stopTime | GeneralizedTime | 1 | This attribute contains the stop date and time of the on-demand session. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute contains the MAC address of the peer MEP. See G.8013 for details. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| oamPduGenerationType | OamPduGenerationType | 1 | This attribute contains the pattern that is used for the generation of OAM PDUs. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type. - support: MANDATORY - condition: none |
| measurementInterval | Integer | 1 | This attribute contains the discrete non overlapping periods of time (in seconds) during which measurements are performed (i.e., OAM messages are generated) and reports are gathered at the end of the measurement intervals. Note that the value 0 means a degenerated measurement interval with a single OAM message and the report is sent as immediately as possible. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: Non-negative - support: MANDATORY - condition: none |
| messagePeriod | MessagePeriod | 1 | This attribute contains the frequency of the OAM message (PDU) generation within a series. Note that the value 0 means that only one OAM message per measurement interval is generated. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See corresponding Enum. - support: MANDATORY - condition: none |
| repetitionPeriod | RepetitionPeriod | 1 | This attribute contains the time between the start of two measurement intervals. This IS applicable for the repetitive instance type and MAY be applicable for the repetitive series type. Note that a value of 0 means not applicable (NA), which is for the cases of single instance, single series, or repetitive series without extra gap in between the measurement intervals (i.e., also as known as continuous series). | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority of the OAM PDU frames. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: 0, 1, 2, 3, 4, 5, 6, 7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of LMM/LMR measurement. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### OnDemandMeasurementJob class

Qualified Name: G.8052_v2.07-model::Object Classes::OnDemandMeasurementJob

Description: Basic attributes: onDemandDualEndedMeasurementJobControlSink, onDemandDualEndedMeasurementJobControlSource, onDemandSingleEndedMeasurementJobControl

Description: This object class models the "proxy" of all OnDemand Measurement Jobs.

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class OnDemandMeasurementJob

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _onDemandDualEndedMeasurementJobControlSink | OnDemandDualEndedMeasurementJobControlSink | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemandSingleEndedMeasurementJobControl | OnDemandSingleEndedMeasurementJobControl | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemandDualEndedMeasurementJobControlSource | OnDemandDualEndedMeasurementJobControlSource | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### OnDemandMeasurementJobControl class

Qualified Name: G.8052_v2.07-model::Object Classes::OnDemandMeasurementJobControl

Description: Measurement configuration related attributes: oamPduGenerationType, startTime, stopTime, messagePeriod, repetitionPeriod, measurementInterval

Description: Optional attributes: dataTlvLength, testIdentifier

Description: Basic attributes: destinationAddress, priority

Description: This object class contains the common attributes representing the configuration of an on-demand measurement job.

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class OnDemandMeasurementJobControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| startTime | GeneralizedTime | 1 | This attribute contains the start date and time of the on-demand session. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| stopTime | GeneralizedTime | 1 | This attribute contains the stop date and time of the on-demand session. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute contains the MAC address of the peer MEP. See G.8013 for details. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| oamPduGenerationType | OamPduGenerationType | 1 | This attribute contains the pattern that is used for the generation of OAM PDUs. | OpenModelAttribute - valueRange: See data type. - support: MANDATORY - condition: none |
| measurementInterval | Integer | 1 | This attribute contains the discrete non overlapping periods of time (in seconds) during which measurements are performed (i.e., OAM messages are generated) and reports are gathered at the end of the measurement intervals. Note that the value 0 means a degenerated measurement interval with a single OAM message and the report is sent as immediately as possible. | OpenModelAttribute - valueRange: Non-negative - support: MANDATORY - condition: none |
| messagePeriod | MessagePeriod | 1 | This attribute contains the frequency of the OAM message (PDU) generation within a series. Note that the value 0 means that only one OAM message per measurement interval is generated. | OpenModelAttribute - valueRange: See corresponding Enum. - support: MANDATORY - condition: none |
| repetitionPeriod | RepetitionPeriod | 1 | This attribute contains the time between the start of two measurement intervals. This IS applicable for the repetitive instance type and MAY be applicable for the repetitive series type. Note that a value of 0 means not applicable (NA), which is for the cases of single instance, single series, or repetitive series without extra gap in between the measurement intervals (i.e., also as known as continuous series). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority of the OAM PDU frames. | OpenModelAttribute - valueRange: 0, 1, 2, 3, 4, 5, 6, 7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of LMM/LMR measurement. | OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### OnDemandSingleEndedMeasurementJobControl class

Qualified Name: G.8052_v2.07-model::Object Classes::OnDemandSingleEndedMeasurementJobControl

Description: Basic attribute: oamType

Description: Performance data related attribute: onDemandPerformanceData

Description: This object class represents an on-demand measurement job controller for 2-way measurements. It is created as a result of an establishOnDemandSingleEndedMeasurementJob() operation. It is deleted either automatically after the measurement job has completed (stop time reached) and the performance data AVC notification has been sent, or by an explicit abortOnDemandMeasurementJob() operation while the measurement job is running.

Description: Performance data related operation: getIntermediateReport

Parent class: OnDemandMeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class OnDemandSingleEndedMeasurementJobControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| oamType | 2WayOamType | 1 | This attribute contains the type of the 2-way measurement. | OpenModelAttribute - valueRange: See corresponding Enum. - support: MANDATORY - condition: none |
| onDemandPerformanceData | OnDemandPerformanceData | 1 | This attribute provides the final (at the end of the measurement interval) PM data report. The OS receives the final PM data report via the attributeValueChangeNotification of this attribute. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| startTime | GeneralizedTime | 1 | This attribute contains the start date and time of the on-demand session. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| stopTime | GeneralizedTime | 1 | This attribute contains the stop date and time of the on-demand session. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute contains the MAC address of the peer MEP. See G.8013 for details. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| oamPduGenerationType | OamPduGenerationType | 1 | This attribute contains the pattern that is used for the generation of OAM PDUs. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type. - support: MANDATORY - condition: none |
| measurementInterval | Integer | 1 | This attribute contains the discrete non overlapping periods of time (in seconds) during which measurements are performed (i.e., OAM messages are generated) and reports are gathered at the end of the measurement intervals. Note that the value 0 means a degenerated measurement interval with a single OAM message and the report is sent as immediately as possible. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: Non-negative - support: MANDATORY - condition: none |
| messagePeriod | MessagePeriod | 1 | This attribute contains the frequency of the OAM message (PDU) generation within a series. Note that the value 0 means that only one OAM message per measurement interval is generated. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See corresponding Enum. - support: MANDATORY - condition: none |
| repetitionPeriod | RepetitionPeriod | 1 | This attribute contains the time between the start of two measurement intervals. This IS applicable for the repetitive instance type and MAY be applicable for the repetitive series type. Note that a value of 0 means not applicable (NA), which is for the cases of single instance, single series, or repetitive series without extra gap in between the measurement intervals (i.e., also as known as continuous series). | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority of the OAM PDU frames. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: 0, 1, 2, 3, 4, 5, 6, 7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of LMM/LMR measurement. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | Inherited from Class OnDemandMeasurementJobControl OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### PhysicalSubsystem class

Qualified Name: G.8052_v2.07-model::Object Classes::PhysicalSubsystem

Description: Basic attributes: _containedTerminationPointList, eui-48Identifier

Description: This object class groups a set of atomic functions within the network element. It is used for the configuration of all properties (e.g., the EUI-48 identifier (MAC address)) that are common to all object instances (and their containing object instances), such as termination points, grouped by the PhysicalSubsystem.
																				Even though these common properties are defined in the PhysicalSubsystem, these properties will still be also part of the individual object classes are read-only in the individual objects if the PhysicalSubsystem is supported; and read-write if the PhysicalSubsystem is not supported.
																				For the MAC Address property, there are two-way pointers between the Physical Subsystem object and the related termination points (TP). Since MEP and MIP are contained in TP, there is no need for any pointers between the Physical Subsystem MOC and the associated MEP/MIP. The MAC Address will still be an attribute of the TP, MEP, and MIP, but read-only if the Physical Subsystem MOC is supported in the Ethernet NE. Otherwise, the MAC address of the TP, MEP, and MIP will be read-write if the Physical Subsystem is not supported.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class PhysicalSubsystem

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _containedTerminationPointList | Tp_Pac | 0..* | This navigable attribute points to the Termination Point objects that have the same common MAC Address defined in this Physical Subsystem object. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| eui-48Identifier | MacAddress | 1 | This attribute provides the configured MAC Address that is common to all the Termination Point objects identified in the _containedTerminationPointList attribute. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ProActiveDualEndedMeasurementJobControlSink class

Qualified Name: G.8052_v2.07-model::Object Classes::ProActiveDualEndedMeasurementJobControlSink

Description: This object class allows the control of the proactive 1-way measurement. It is created as a result of an establishProActiveDualEndedMeasurementJobSink() operation. Lifecycle: A pre-condition of deleting the object is that the "Enable" attribute should have the value "FALSE".

Description: Basic attributes: oamType, isEnabled, sourceAddress

Description: Optional attribute: testIdentifier

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProActiveDualEndedMeasurementJobControlSink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isEnabled | Boolean | 1 | This attribute identifies the state of the measurement job. If set to TRUE, the MEP performs proactive Performance Measurement. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| sourceAddress | MacAddress | 1 | This attribute contains the MAC address of the peer MEP. See G.8013 for details. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. | OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| oamType | 1WayOamType | 1 | This attribute contains the type of the 1way measurement. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactive1SlCurrentData | Proactive1SlCurrentData | 2..3 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _proactive1DmCurrentData | Proactive1DmCurrentData | 2..3 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ProActiveDualEndedMeasurementJobControlSource class

Qualified Name: G.8052_v2.07-model::Object Classes::ProActiveDualEndedMeasurementJobControlSource

Description: This object class represents a proactive measurement job controller source for 1way measurements. It is created as a result of an establishProactiveDualEndedMeasurementJobSource() operation.

Description: Basic attribue: oamType

Parent class: ProActiveMeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProActiveDualEndedMeasurementJobControlSource

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| oamType | 1WayOamType | 1 | This attribute contains the type of the 1way measurement. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isEnabled | Boolean | 1 | This attribute identifies the state of the measurement job. If set to TRUE, the MEP performs proactive Performance Measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute provides the Unicast MAC address of the intented destination. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| period | OamPeriod | 1 | This attribute indicates the period (frequency) of the measurement frame transmission. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 100ms, 1s, 10s - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority value on which the MEP performs the measurement. When the measurement is enabled, the MEP should use this value to encode the priority of generated measurement frames. The EMF usese this value to assign the "P" parameter of the measurement operation. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of 2-way loss measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ProActiveMeasurementJob class

Qualified Name: G.8052_v2.07-model::Object Classes::ProActiveMeasurementJob

Description: This object class models the "proxy" of all ProActive Measurement Jobs.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProActiveMeasurementJob

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _proActiveDualEndedMeasurementJobControlSink | ProActiveDualEndedMeasurementJobControlSink | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _proActiveSingleEndedMeasurementJobControl | ProActiveSingleEndedMeasurementJobControl | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _proActiveDualEndedMeasurementJobControlSource | ProActiveDualEndedMeasurementJobControlSource | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### ProActiveMeasurementJobControl class

Qualified Name: G.8052_v2.07-model::Object Classes::ProActiveMeasurementJobControl

Description: Basic attributes: destinationAddress, isEnabled, period, priority

Description: This object class contains the common attributes representing the configuration of an Proactive Measurement Job.

Description: Optional attributes: dataTlvLength, testIdentifier

Abstract class

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProActiveMeasurementJobControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isEnabled | Boolean | 1 | This attribute identifies the state of the measurement job. If set to TRUE, the MEP performs proactive Performance Measurement. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute provides the Unicast MAC address of the intented destination. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| period | OamPeriod | 1 | This attribute indicates the period (frequency) of the measurement frame transmission. | OpenModelAttribute - valueRange: 100ms, 1s, 10s - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority value on which the MEP performs the measurement. When the measurement is enabled, the MEP should use this value to encode the priority of generated measurement frames. The EMF usese this value to assign the "P" parameter of the measurement operation. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of 2-way loss measurement. | OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ProActiveSingleEndedMeasurementJobControl class

Qualified Name: G.8052_v2.07-model::Object Classes::ProActiveSingleEndedMeasurementJobControl

Description: Basic attributes: oamType, _proActiveCcmSinkAttributeList

Description: This object class represents a proactive measurement job controller for 2-way measurements. It is created as a result of an establishProActiveSingleEndedMeasurementJob() operation.

Parent class: ProActiveMeasurementJobControl

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProActiveSingleEndedMeasurementJobControl

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| oamType | 2WayOamType | 1 | This attribute contains the type of the 2-way measurement. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proActiveCcmSinkAttributeList | DegradedDetection_Pac | 0..1 | This relationship provides the configuration parameters for detecting "degraded signal" (DEG). Only applicable if the OAM Type is set to LM CCM. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactiveDmCurrentData | ProactiveDmCurrentData | 2..3 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _proactiveLmCurrentData | ProactiveLmCurrentData | 2..3 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| isEnabled | Boolean | 1 | This attribute identifies the state of the measurement job. If set to TRUE, the MEP performs proactive Performance Measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| destinationAddress | MacAddress | 1 | This attribute provides the Unicast MAC address of the intented destination. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| period | OamPeriod | 1 | This attribute indicates the period (frequency) of the measurement frame transmission. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 100ms, 1s, 10s - support: MANDATORY - condition: none |
| priority | Integer | 1 | This attribute contains the priority value on which the MEP performs the measurement. When the measurement is enabled, the MEP should use this value to encode the priority of generated measurement frames. The EMF usese this value to assign the "P" parameter of the measurement operation. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| testIdentifier | Integer | 1 | This attribute is used to distinguish each measurement session if multiple measurement sessions are simultaneously activated towards a peer MEP including concurrent on-demand and proactive tests. It must be unique at least within the context of any measurement type for the MEG and initiating MEP. Note: The attribute is not used in case of 2-way loss measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: 0..(2^32) - 1 - support: OPTIONAL - condition: none |
| dataTlvLength | Integer | 1 | This parameter provides the size of the optional data TLV. Non-negative integer represents the number of bytes for the length of the padding TLV. Notes: When configuring this parameter one should be aware of the maximum allowed total frame size limitation. The attribute is not used in case of 2-way loss measurement. | Inherited from Class ProActiveMeasurementJobControl OpenModelAttribute - valueRange: Depends on the allowed MTU size. - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### Proactive1DmCurrentData class

Qualified Name: G.8052_v2.07-model::Object Classes::Proactive1DmCurrentData

Description: Basic attribute: timestamp

Description: This object class represents the PM current data collected in a pro-active delay measurement job (using 1DM).

Description: Performance data related attribute: delayMeasurementParameters

Parent class: CurrentData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Proactive1DmCurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| timestamp | GeneralizedTime | 1 | This attribute contains the interval start time. The value is bound to the quarter of an hour in case of a 15 minute interval and bound to the hour in case of a 24 hour interval. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| delayMeasurementParameters | ProActive1DmPerformanceParameters | 1 | This attribute contains the current pro-active delay measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactive1DmHistoryData | Proactive1DmHistoryData | 1..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _associatedThresholdProfileList | ThresholdProfile | 0..* |  | Inherited from Class CurrentData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### Proactive1DmHistoryData class

Qualified Name: G.8052_v2.07-model::Object Classes::Proactive1DmHistoryData

Description: Performance data related attribute: delayMeasurementParameters

Description: This object class represents the PM history data collected in a pro-active delay measurement job (using 1DM).

Description: Basic attribute: isSuspectIntervalFlagSet

Parent class: «Q.822» HistoryData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Proactive1DmHistoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isSuspectIntervalFlagSet | Boolean | 1 | This attribute indicates that the performance data may not be reliable. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| delayMeasurementParameters | ProActive1DmPerformanceParameters | 1 | This attribute contains the history pro-active delay measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| historyDataId |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| periodEndTime |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatameasurementlistpkg | «Q.822» historyDataMeasurementListPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatasuspectintervalflagpkg | «Q.822» historyDataSuspectIntervalFlagPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» objectdeletenotificationpkg | «Q.822» objectDeleteNotificationPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### Proactive1SlCurrentData class

Qualified Name: G.8052_v2.07-model::Object Classes::Proactive1SlCurrentData

Description: Basic attribute: timestamp

Description: Performance data related attribute: lossMeasurementParameters

Description: This object class represents the PM current data collected in a pro-active loss measurement job (using 1SL).

Parent class: CurrentData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Proactive1SlCurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| timestamp | GeneralizedTime | 1 | This attribute indicates the start of the current monitoring interval. The value is bound to the quarter of an hour in case of a 15 minute interval and bound to the hour in case of a 24 hour interval. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lossMeasurementParameters | ProActive1SlPerformanceParameters | 1 | This attribute contains the current pro-active loss measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactive1SlHistoryData | Proactive1SlHistoryData | 1..* |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _associatedThresholdProfileList | ThresholdProfile | 0..* |  | Inherited from Class CurrentData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### Proactive1SlHistoryData class

Qualified Name: G.8052_v2.07-model::Object Classes::Proactive1SlHistoryData

Description: Performance data related attribute: lossMeasurementParameters

Description: This object class represents the PM history data collected in a pro-active loss measurement job (using 1SL).

Description: Basic attribute: isSuspectIntervalFlagSet

Parent class: «Q.822» HistoryData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class Proactive1SlHistoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isSuspectIntervalFlagSet | Boolean | 1 | This attribute indicates that the performance data may not be reliable. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lossMeasurementParameters | ProActive1SlPerformanceParameters | 1 | This attribute contains the near end pro-active PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| historyDataId |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| periodEndTime |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatameasurementlistpkg | «Q.822» historyDataMeasurementListPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatasuspectintervalflagpkg | «Q.822» historyDataSuspectIntervalFlagPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» objectdeletenotificationpkg | «Q.822» objectDeleteNotificationPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProactiveDmCurrentData class

Qualified Name: G.8052_v2.07-model::Object Classes::ProactiveDmCurrentData

Description: Basic attribute: timestamp

Description: This object class represents the PM current data collected in a pro-active delay measurement job (using DMM/DMR).

Description: Performance data related attribute: delayMeasurementParameters

Parent class: CurrentData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProactiveDmCurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| timestamp | GeneralizedTime | 1 | This attribute contains the interval start time. The value is bound to the quarter of an hour in case of a 15 minute interval and bound to the hour in case of a 24 hour interval. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| delayMeasurementParameters | ProActiveDmPerformanceParameters | 1 | This attribute contains the current pro-active delay measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactiveDmHistoryData | ProactiveDmHistoryData | 1..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _associatedThresholdProfileList | ThresholdProfile | 0..* |  | Inherited from Class CurrentData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProactiveDmHistoryData class

Qualified Name: G.8052_v2.07-model::Object Classes::ProactiveDmHistoryData

Description: Performance data related attribute: delayMeasurementParameters

Description: This object class represents the PM history data collected in a pro-active delay measurement job (using DMM/DMR).

Description: Basic attribute: isSuspectIntervalFlagSet

Parent class: «Q.822» HistoryData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProactiveDmHistoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isSuspectIntervalFlagSet | Boolean | 1 | This attribute indicates that the performance data may not be reliable. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| delayMeasurementParameters | ProActiveDmPerformanceParameters | 1 | This attribute contains the history pro-active delay measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| historyDataId |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| periodEndTime |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatameasurementlistpkg | «Q.822» historyDataMeasurementListPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatasuspectintervalflagpkg | «Q.822» historyDataSuspectIntervalFlagPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» objectdeletenotificationpkg | «Q.822» objectDeleteNotificationPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProactiveLmCurrentData class

Qualified Name: G.8052_v2.07-model::Object Classes::ProactiveLmCurrentData

Description: Performance data related attribute: lossMeasurementParameters

Description: Basic attribute: timestamp

Description: This object class represents the PM current data collected in a pro-active loss measurement job (using LMM/LMR or SLM/SLR).

Parent class: CurrentData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProactiveLmCurrentData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| timestamp | GeneralizedTime | 1 | This attribute indicates the start of the current monitoring interval. The value is bound to the quarter of an hour in case of a 15 minute interval and bound to the hour in case of a 24 hour interval. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lossMeasurementParameters | ProActiveLmPerformanceParameters | 1 | This attribute contains the pro-active loss measurement PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _proactiveLmHistoryData | ProactiveLmHistoryData | 1..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| _associatedThresholdProfileList | ThresholdProfile | 0..* |  | Inherited from Class CurrentData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| suspectIntervalFlag | Boolean | 1 | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: – Suspect data were detected by the actual resource doing data collection. – Transition of the administrativeState attribute to/from the 'lock' state. – Transition of the operationalState to/from the 'disabled' state. – Scheduler setting that inhibits the collection function. – The performance counters were reset during the interval. – The currentData (or subclass) object instance was created during the monitoring period. | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| elapsedTime |  | 1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» filtersuppressionpkg | «Q.822» filterSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historyretentionpkg | «Q.822» historyRetentionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» maxsuppressedintervalspkg | «Q.822» maxSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» measurementlistpkg | «Q.822» measurementListPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» scheduledpmreportpkg | «Q.822» scheduledPMReportPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» thresholdpkg | «Q.822» thresholdPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» zerosuppressionpkg | «Q.822» zeroSuppressionPkg | 0..1 |  | Inherited from Class «Q.822» CurrentData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProactiveLmHistoryData class

Qualified Name: G.8052_v2.07-model::Object Classes::ProactiveLmHistoryData

Description: This object class represents the PM history data collected in a pro-active loss measurement job (using LMM/LMR or SLM/SLR).

Description: Performance data related attribute: lossMeasurementParameters

Description: Basic attribute: isSuspectIntervalFlagSet

Parent class: «Q.822» HistoryData

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ProactiveLmHistoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| isSuspectIntervalFlagSet | Boolean | 1 | This attribute indicates that the performance data may not be reliable. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lossMeasurementParameters | ProActiveLmPerformanceParameters | 1 | This attribute contains the near and far end pro-active PM data. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| historyDataId |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| periodEndTime |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| granularityPeriod |  | 1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» numsuppressedintervalspkg | «Q.822» numSuppressedIntervalsPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatameasurementlistpkg | «Q.822» historyDataMeasurementListPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» historydatasuspectintervalflagpkg | «Q.822» historyDataSuspectIntervalFlagPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» observedmanagedobjectpkg | «Q.822» observedManagedObjectPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| «q.822» objectdeletenotificationpkg | «Q.822» objectDeleteNotificationPkg | 0..1 |  | Inherited from Class «Q.822» HistoryData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### RapsCapableHalfMipBidirectional class

Qualified Name: G.8052_v2.07-model::Object Classes::RapsCapableHalfMipBidirectional

Description: This object class models the MIP Sink and MIP Source functions. Only the bidirectional MIP is defined in the model.

Description: Basic attributes: mel, mipMac, rapsPriority

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class RapsCapableHalfMipBidirectional

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mipMac | MacAddress | 1 | This attribute contains the MAC address of the MIP instance. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MIP. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| rapsPriority | Integer | 1 | This attribute models the MI_RAPS_Pri signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### ThresholdProfile class

Qualified Name: G.8052_v2.07-model::Object Classes::ThresholdProfile

Description: This object class defines the PM threshold settings in the network element.
																																				Multiple ThresholdProfile instances may be created to support, e.g., different grade of services.
																																				These profiles are associated to the Current Data object instances modelling the current registers via a pointer from the Current Data instances.

Description: 1LM threshold related attribute: 1LmThresholdList

Description: 1DM threshold related attribute: 1DmThresholdList

Description: DM threshold related attribute: dmThresholdList

Description: LM threshold related attribute: lmThresholdList

Description: Basic attribute: _associatedCurrentDataList

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class ThresholdProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| dmThresholdList | DmThresholdValues | 0..* | This attribute contains the threshold settings for the delay measurements (2-way) in the current registers. This attribute is defined as a list because it may contain lists for different granularity periods (e.g., 24hr, 15min, ...). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| 1DmThresholdList | 1DmThresholdValues | 0..* | This attribute contains the threshold settings for the delay measurements (1-way) in the current registers. This attribute is defined as a list because it may contain lists for different granularity periods (e.g., 24hr, 15min, ...). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lmThresholdList | LmThresholdValues | 0..* | This attribute contains the threshold settings for the frame loss measurements (2-way) in the current registers. This attribute is defined as a list because it may contain lists for different granularity periods (e.g., 24hr, 15min, ...). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| 1LmThresholdList | 1LmThresholdValues | 0..* | This attribute contains the threshold settings for the frame loss measurements (1-way) in the current registers. This attribute is defined as a list because it may contain lists for different granularity periods (e.g., 24hr, 15min, ...). | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| _associatedCurrentDataList | CurrentData | 0..* | This navigable end contains the list of Current Data object instances associated to the Threshold Profile. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| objectClass | ObjectIdentifier | 1 | ObjectClass ::= CHOICE{ globalForm [0] OBJECT IDENTIFIER, localForm [1] INTEGER} | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nameBinding | ObjectIdentifier | 1 |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| packages | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| allomorphs | ObjectIdentifier | 0..* |  | Inherited from Class «X.721» Top OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

### Associations

## OperationExceptions

Qualified Name: G.8052_v2.07-model::OperationExceptions

### CommonExceptions

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions

#### Diagrams

Figure 1 Diagram Common operations exceptions

#### Datatypes

##### ALREADY_IN_POST_CONDITION datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::ALREADY_IN_POST_CONDITION

Description: Raised when the the server is already in the post-condition. It can be used by operations which are not defined as idempotent.

Table 1 Attributes for Data Type ALREADY_IN_POST_CONDITION

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### CAPACITY_EXCEEDED datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::CAPACITY_EXCEEDED

Description: Raised when the request will result in resources being created or activated beyond the capacity supported by the NE.

Table 1 Attributes for Data Type CAPACITY_EXCEEDED

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### DUPLICATE datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::DUPLICATE

Description: Raised when an entity cannot be created because an object with the same identifier/name already exists.

Table 1 Attributes for Data Type DUPLICATE

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### ENTITY_NOT_FOUND datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::ENTITY_NOT_FOUND

Description: Raised when at least one of the specified entities does not exist.

Table 1 Attributes for Data Type ENTITY_NOT_FOUND

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### LOW_PRIORITY datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::LOW_PRIORITY

Description: Raised when the operation cannot be excecuted beacuse of an other higher priority command is still in effect.

Table 1 Attributes for Data Type LOW_PRIORITY

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### NOT_IN_VALID_STATE datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::NOT_IN_VALID_STATE

Description: Raised when the state of the specified object is such that the NE cannot perform the request. In other words, the environment or the application is not in an appropriate state for the requested operation.

Table 1 Attributes for Data Type NOT_IN_VALID_STATE

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### OBJECT_IN_USE datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::CommonExceptions::OBJECT_IN_USE

Description: Raised when the object identified in the request is currently in use.

Table 1 Attributes for Data Type OBJECT_IN_USE

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

### GenericExceptions

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions

#### Diagrams

Figure 1 Diagram Generic exceptions

#### Datatypes

##### ACCESS_DENIED datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::ACCESS_DENIED

Description: Raised when the EMS does not have access rights to request the given operation.

Table 1 Attributes for Data Type ACCESS_DENIED

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### COMM_LOSS datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::COMM_LOSS

Description: Raised when the NE is unable to communicate with an underlying system or resource, and such communication is required to complete the request.

Table 1 Attributes for Data Type COMM_LOSS

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### INTERNAL_ERROR datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::INTERNAL_ERROR

Description: Raised when the NE has an internal error.

Table 1 Attributes for Data Type INTERNAL_ERROR

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### INVALID_INPUT datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::INVALID_INPUT

Description: Raised when the request contains an input parameter that is syntactically incorrect or identifies an object of the wrong type or is out of range (as defined in the model or because of NE limitation).

Table 1 Attributes for Data Type INVALID_INPUT

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### NOT_IMPLEMENTED datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::NOT_IMPLEMENTED

Description: Raised when the entire request is not supported by the NE or the request with the specified input parameters is not supported.

Table 1 Attributes for Data Type NOT_IMPLEMENTED

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

##### UNABLE_TO_COMPLY datatype

Qualified Name: G.8052_v2.07-model::OperationExceptions::GenericExceptions::UNABLE_TO_COMPLY

Description: Raised when the NE cannot perform the request. Use Cases may identify specific conditions that will result in this exception.

Table 1 Attributes for Data Type UNABLE_TO_COMPLY

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| errorReason | String | 1 | This Parameter is a string indicating further details about the exception. It is a free format string filled by the NE (i.e., the Server of the Operation). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

## Pacs

Qualified Name: G.8052_v2.07-model::Pacs

Description: Note that in the UML modeling, a "_package" object class is used to group a set of attributes/parameters and parameters for the purpose that they can be conditionally associated to an object class.

### Classes

#### CsfConfigSource_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::CsfConfigSource_Pac

Description: This package provides the CSF related MI signals defined in the adaptation source atomic functions.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class CsfConfigSource_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| csfConfig | CsfConfig | 1 | This attribute models the combination of all CSF related MI signals (MI_CSF_Enable, MI_CSFrdifdi_Enable, MI_CSFdci_Enable) as defined in G.8021. | OpenModelAttribute - valueRange: true, false - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### CsfRdiFdiEnableSink_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::CsfRdiFdiEnableSink_Pac

Description: This pac provides the client layer related MI attributes defined in the following adaptation functions: G.8021 Sn/ETH, Sn-X-L/ETH, Sm/ETH, Sm-X-L/ETH, Pq/ETH, Pq-X-L/ETH, ODUkP/ETH.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class CsfRdiFdiEnableSink_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| csfRdiFdiEnable | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable information defined in G.8021. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### CsfRdiFdiEnableSource_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::CsfRdiFdiEnableSource_Pac

Description: This pac provides the client layer related MI attributes defined in the following adaptation functions: G.8021 Sn/ETH, Sn-X-L/ETH, Sm/ETH, Sm-X-L/ETH, Pq/ETH, Pq-X-L/ETH, ODUkP/ETH.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class CsfRdiFdiEnableSource_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| csfRdiFdiEnable | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable information defined in G.8021. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### CsfReportSink_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::CsfReportSink_Pac

Description: This package provides the client layer related MI attributes defined in the following adaptation functions: G.8021 Sn/ETH, Sn-X-L/ETH, Sm/ETH, Sm-X-L/ETH, Pq/ETH, Pq-X-L/ETH, ODUkP/ETH, Sn-X/ETC3, ODU2P/ETHPP-OS.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class CsfReportSink_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| csfReport | Boolean | 1 | This attribute models the MI_CSF_Reported information defined in G.8021. | OpenModelAttribute - valueRange: true, false - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### DegradedDetection_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::DegradedDetection_Pac

Description: This package contains the configuration parameters for detecting "degraded signal" (DEG).

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class DegradedDetection_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| lmDegm | Integer | 1 | This attribute defines the number of consecutive bad seconds necessary for the "degraded" detection. See also section "Degraded signal defect (dDEG)" in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lmDegThr | Integer | 1 | This attribute defines the threshold for declaring a "bad second". See also section "Degraded signal defect (dDEG)" in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| lmM | Integer | 1 | This attribute defines the number of consecutive good seconds necessary for the clearing of "degraded". See also section "Degraded signal defect (dDEG)" in G.8021. | OpenModelAttribute - valueRange: 2..10 - support: MANDATORY - condition: none |
| lmTfMin | Integer | 1 | This attribute defines the necessary number of transmitted frames to enable the detection of "bad seconds". See also section "Degraded signal defect (dDEG)" in G.8021. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### EthServerTtpSink_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::EthServerTtpSink_Pac

Description: This pac models the server layer (i.e. ETHx) related function of ETHx/ETH-m_A_Sk defined in 9.3.3/G.8021. This pac is contained in the ETH_TrailTerminationPointSink object.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class EthServerTtpSink_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| adminState | AdminState | 1 | This attribute models the ETHx/ETH-m_A_Sk_MI_Admin_State information defined in G.8021. | OpenModelAttribute - valueRange: Lock, Normal - support: MANDATORY - condition: none |
| priorityRegenerate | PriorityMapping | 1 | This attribute models the ETHx/ETH-m _A_Sk_MI_P_Regenerate information defined in G.8021. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portVid | Integer | 1 | This attribute models the ETHx/ETH-m _A_Sk_MI_PVID information defined in G.8021. | OpenModelAttribute - valueRange: 0..4095 - support: MANDATORY - condition: none |
| priorityCodePointConfig | PcpCoding | 1 | This attribute models the ETHx/ETH-m _A_Sk_MI_PCP_Config information defined in G.8021. | OpenModelAttribute - valueRange: see Enumeration - support: MANDATORY - condition: none |
| etherType | VlanType | 1 | This attribute models the ETHx/ETH-m _A_Sk_MI_Etype information defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| frametypeConfig | FrameType | 1 | This attribute models the ETHx/ETH-m_A_Sk_MI_Frametype_Config information defined in G.8021. | OpenModelAttribute - valueRange: see Enumeration - support: MANDATORY - condition: none |
| filterConfig | MacAddress | 1 | This attribute models the ETHx/ETH-m_A_Sk_MI_Filter_Config information defined in G.8021. It indicates the configured filter action for each of the 33 group MAC addresses for control frames. The 33 MAC addresses are: 01-80-C2-00-00-10, 01-80-C2-00-00-00 to 01-80-C2-00-00-0F, and 01-80-C2-00-00-20 to 01-80-C2-00-00-2F. The filter action is Pass or Block. If the destination address of the incoming ETH_CI_D matches one of the above addresses, the filter process shall perform the corresponding configured filter action. If none of the above addresses match, the ETH_CI_D is passed. | OpenModelAttribute - valueRange: MacAddress: 01-80-C2-00-00-10, 01-80-C2-00-00-00 to 01-80-C2-00-00-0F, and 01-80-C2-00-00-20 to 01-80-C2-00-00-2F; ActionEnum: PASS, BLOCK - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### EthServerTtpSource_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::EthServerTtpSource_Pac

Description: This pac models the server layer (i.e. ETHx) related function of ETHx/ETH-m_A_So defined in 9.3.3/G.8021.
																																								This pac is contained in the ETH_TrailTerminationPointSource object.

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class EthServerTtpSource_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| adminState | AdminState | 1 | This attribute models the ETHx/ETH-m_A_So_MI_Admin_State information defined in G.8021. | OpenModelAttribute - valueRange: Lock, Normal - support: MANDATORY - condition: none |
| etherType | VlanType | 1 | This attribute models the ETHx/ETH-m_A_So_MI_Etype information defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| priorityCodePointConfig | PcpCoding | 1 | This attribute models the ETHx/ETH-m _A_Sk_MI_PCP_Config information defined in G.8021. | OpenModelAttribute - valueRange: see Enumeration - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### FilterConfigSink_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::FilterConfigSink_Pac

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class FilterConfigSink_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| filterConfig | MacAddress | 1 | This attribute models the FilteConfig MI defined in 8.3/G.8021. It indicates the configured filter action for each of the 33 group MAC addresses for control frames. The 33 MAC addresses are: 01-80-C2-00-00-10, 01-80-C2-00-00-00 to 01-80-C2-00-00-0F, and 01-80-C2-00-00-20 to 01-80-C2-00-00-2F. The filter action is Pass or Block. If the destination address of the incoming ETH_CI_D matches one of the above addresses, the filter process shall perform the corresponding configured filter action. If none of the above addresses match, the ETH_CI_D is passed. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| macLength | Integer | 1 | This attribute models the MAC_Lenght MI defined in 8.6/G.8021 for the MAC Length Check process. It indicates the allowed maximum frame length in bytes. | OpenModelAttribute - valueRange: 1518, 1522, 2000 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### Tp_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::Tp_Pac

Abstract class

Applied Stereotypes:

- OpenModelClass

  - support: 
       MANDATORY

Table 1 Attributes for Class Tp_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| alarmStatus |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| crossConnectionObjectPointer |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| currentProblemList |  | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| alarmSeverityAssignmentProfilePointer |  | 1 | This role (when present) represents the Termination Point (include its child classes Trail/Connection Termination Point Source/Sink/Bidirectional, Network CTP/TTP Source/Sink/Bidirectional) capability to identify the Alarm Severity Assignment Profile that is associated with this Termination Point. When this role is present, the TerminationPoint.alarmSeverityAssignmentProfilePointe shall carry the DN of Alarm Severity Assignment Profile. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |
| _containingPhysicalSubsystem | PhysicalSubsystem | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### VlanConfigSink_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::VlanConfigSink_Pac

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class VlanConfigSink_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vlanConfig | Integer | 1 | This attribute models the ETHx/ETH-m_A_So_MI_Vlan_Config information defined in G.8021. | OpenModelAttribute - valueRange: -1, 0, 1..4094 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### VlanConfigSource_Pac class

Qualified Name: G.8052_v2.07-model::Pacs::VlanConfigSource_Pac

Parent class: G8052LocalClass

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

Table 1 Attributes for Class VlanConfigSource_Pac

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vlanConfig | Integer | 1 | This attribute models the ETHx/ETH-m_A_So_MI_Vlan_Config information defined in G.8021. | OpenModelAttribute - valueRange: -1, 0, 1..4094 - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

## Server/Client Object Classes

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes

### Classes

#### ATM_ConnectionTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::ATM_ConnectionTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### MT_ConnectionTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::MT_ConnectionTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### MT_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::MT_TrailTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### OTH_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::OTH_TrailTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### PDH_ConnectionTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::PDH_ConnectionTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### PDH_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::PDH_TrailTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

#### SDH_TrailTerminationPoint class

Qualified Name: G.8052_v2.07-model::Server/Client Object Classes::SDH_TrailTerminationPoint

Description: Not in scope of G.8052.

Abstract class

Applied Stereotypes:

- OpenModelClass

  - condition: 
       none

  - support: 
       MANDATORY

### Associations

## Type Definitions Common

Qualified Name: G.8052_v2.07-model::Type Definitions Common

Description: This package contains all type definitions that are common to all UML interface specifications in SG 15.

### Datatypes

#### BitString primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Common::BitString

Description: This primitive type defines a bit oriented string.
																																								The size of the BitString will be defined in the valueRange property of the attribute; according to ASN.1 (X.680).
																																								The semantic of each bit position will be defined in the Documentation field of the attribute.

#### DateAndTime primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Common::DateAndTime

Description: This primitive type defines the date and time according to the following structure:
																																								"yyyyMMddhhmmss.s[Z|{+|-}HHMm]" where:
																																								yyyy	"0000".."9999"	year
																																								MM		"01".."12"			month
																																								dd		"01".."31"			day
																																								hh		"00".."23"			hour
																																								mm		"00".."59"			minute
																																								ss		"00".."59"			second
																																								s		".0"..".9"			tenth of second (set to ".0" if EMS or NE cannot support this granularity)
																																								Z		"Z"					indicates UTC (rather than local time)
																																								{+|-}	"+" or "-"			delta from UTC
																																								HH		"00".."23"			time zone difference in hours
																																								Mm		"00".."59"			time zone difference in minutes.

#### Real primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Common::Real

Description: This primitive type maps to the "realnumber" defined in Recommendation X.680.

## Type Definitions Specific

Qualified Name: G.8052_v2.07-model::Type Definitions Specific

Description: This package contains all type definitions that are specific to this Ethernet interface specification.

### Datatypes

#### 1DmThresholdValues datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::1DmThresholdValues

Description: This data type contains the threshold values for frame delay related 1-way measurements.

Table 1 Attributes for Data Type 1DmThresholdValues

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| granularityPeriod | TimePeriod | 1 | This attribute defines the granularity period of the current registers to which the thresholds are assigned. | OpenModelAttribute - valueRange: 24hr, 15min, 1 - 14 min - support: MANDATORY - condition: none |
| nearEndCrossThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the near end cross threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nearEndClearThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the near end clear threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### 1LmThresholdValues datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::1LmThresholdValues

Description: This data type contains the threshold values for frame loss related 1-way measurements.

Table 1 Attributes for Data Type 1LmThresholdValues

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| granularityPeriod | TimePeriod | 1 | This attribute defines the granularity period of the current registers to which the thresholds are assigned. | OpenModelAttribute - valueRange: 24hr, 15min, 1 - 14 min - support: MANDATORY - condition: none |
| nearEndCrossThreshold | StatisticalLmPerformanceParameters | 1 | This attribute contains the near end cross threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nearEndClearThreshold | StatisticalLmPerformanceParameters | 1 | This attribute is only valid for frame loss ratio parameters and counter type parameters working in the "standing condition method" (see G.7710, section 10.1.7.2: Threshold reporting) and contains the near end clear threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### 1WayOamType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::1WayOamType

Contains Enumeration Literals:

- LM_CCM

- 1DM

- 1SL

#### 2WayOamType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::2WayOamType

Description: This enumeration contains OAM types that produces 2-way measurement, such as Near-end and Far-end loss measurement, Near-end and Far-end delay measurement and round-trip delay measurement.

Contains Enumeration Literals:

- LM_LMM

- DM

- SL

#### AddressTableConfig datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AddressTableConfig

Description: This data type contains the information to be configured in the address table.

Table 1 Attributes for Data Type AddressTableConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| ageing | Integer | 1 | This attribute is used to provision the Ageing time period for entries configured from the Learning Process. The value defines units of seconds. | OpenModelAttribute - valueRange: 10–1 000 000 s - support: MANDATORY - condition: none |
| connectionType | ConnectionType | 1 | This attribute contains the type of the cross connection. TBD in G.8021. Allowed values: - twoPort-point-to-point (unidirectional, no learning) - twoPort-point-to-point (bidirectional, no learning) - nPort(n>2)-point-to-multipoint (unidirectional, no learning) - nPort(n>2)-rooted-multipoint (bidirectional, learning) - nPort(n>2)-multipoint -to-multipoint (bidirectional, learning) | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| isLearningEnabled | Boolean | 1 | This attribute configures whether the learned address information will be used to update the address table (true) or not (false). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### AddressTableInfo datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AddressTableInfo

Table 1 Attributes for Data Type AddressTableInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| addressTableConfig | AddressTableConfig | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| sharedVlanAddressTableReference | SharedVlanAddressTableReference | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| addressTableReference | AddressTableReference | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### AddressTableReference datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AddressTableReference

Description: This data type contains the identifier of the address table that will be copied into a new address table.

Table 1 Attributes for Data Type AddressTableReference

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| tableReference | ObjectIdentifier | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### AddressTuple datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AddressTuple

Description: This data type contains an address tuple consisting of a MAC address and a corresponding port list.

Table 1 Attributes for Data Type AddressTuple

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| address | MacAddress | 1 | This attribute contains the MAC address of the address tuple. | OpenModelAttribute - valueRange: - support: MANDATORY - condition: none |
| portList |  | 0..* | This attribute contains the ports associated to the MAC address in the address tuple. | OpenModelAttribute - valueRange: - support: MANDATORY - condition: none |

#### AdminState enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AdminState

Contains Enumeration Literals:

- LOCK

- NORMAL

#### AggregatorProblemList enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AggregatorProblemList

Description: This enumeration defines the valid list of problems for the LAG aggregator.

Contains Enumeration Literals:

- SSF

- PLL

- TLL

#### AuxiliaryFunctions datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::AuxiliaryFunctions

Description: This choice data type represents a MEP, MIP, or TCS object.

Applied Stereotypes:

- Choice

Table 1 Attributes for Data Type AuxiliaryFunctions

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mip | MipBidirectional | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| mep | Mep | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| trafficConditioning | ETH_TrafficConditioning | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| trafficShaping | ETH_TrafficShaping | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| localId | String | 1 | An identifier that is unique in the context of some scope that is less than the global scope. (consider in the context of Identifier: A property of an entity/role with a value that is unique within an identifier space, where the identifier space is itself unique, and immutable. The identifier therefore represents the identity of the entity/role. An identifier carries no semantics with respect to the purpose of the entity.) | Inherited from Class G8052LocalClass OpenModelAttribute - valueRange: no range constraint - support: MANDATORY |

#### BandwidthReport datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::BandwidthReport

Description: Data type for the bandwidth report.

Table 1 Attributes for Data Type BandwidthReport

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sourceMacAddress | MacAddress | 1 | The sourceMacAddress is the address from the far end. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| portId | Integer | 1 | This attribute returns the far end port identifier. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| nominalBandwidth | Integer | 1 | This attribute returns the configured bandwidth | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| currentBandwidth | Integer | 1 | This attribute returns the current bandwidth. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### ColourMode enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ColourMode

Contains Enumeration Literals:

- COLOUR_BLIND

- COLOUR_AWARE

#### ConnectionType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ConnectionType

Description: This enumeration provides the type of a cross connection.

Contains Enumeration Literals:

- 2_PORT_POINT_TO_POINT_UNIDIRECTIONAL

- 2_PORT_POINT_TO_POINT_BIDIRECTIONAL

- N_PORT_POINT_TO_MULTIPOINT

- N_PORT_ROOTED_TO_MULTIPOINT

- N_PORT_MULTIPOINT_TO_MULTIPOINT

#### ControlFrameFilter datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ControlFrameFilter

Description: This data type identifies the filter action for each of the 33 group MAC addresses (control frames).
																																								Value "false" means block: The frame is discarded by the filter process.
																																								Value "true" means pass: The frame is passed unchanged through the filter process.

Table 1 Attributes for Data Type ControlFrameFilter

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-10 | Boolean | 1 | This attribute identifies the "All LANs Bridge Management Group Address". | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-00 | Boolean | 1 | This attribute identifies the STP/RSTP/MSTP protocol address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-01 | Boolean | 1 | This attribute identifies the IEEE MAC-specific Control Protocols group address (PAUSE protocol). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-02 | Boolean | 1 | This attribute identifies the IEEE 802.3 Slow_Protocols_Multicast address (LACP/LAMP or Link OAM protocols). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-03 | Boolean | 1 | This attribute identifies the Nearest non-TPMR Bridge group address (Port Authentication protocol). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-04 | Boolean | 1 | This attribute identifies the IEEE MAC-specific Control Protocols group address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-05 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-06 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-07 | Boolean | 1 | This attribute identifies the Metro Ethernet Forum E-LMI protocol group address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-08 | Boolean | 1 | This attribute identifies the Provider Bridge Group address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-09 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0A | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0B | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0C | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0D | Boolean | 1 | This attribute identifies the Provider Bridge MVRP address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0E | Boolean | 1 | This attribute identifies the Individual LAN Scope group address, Nearest Bridge group address (LLDP protocol). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-0F | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-20 | Boolean | 1 | This attribute identifies the Customer and Provider Bridge MMRP address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-21 | Boolean | 1 | This attribute identifies the Customer Bridge MVRP address. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-22 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-23 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-24 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-25 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-26 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-27 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-28 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-29 | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2A | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2B | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2C | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2D | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2E | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| 01-80-C2-00-00-2F | Boolean | 1 | Reserved for future standardization. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ControlProcessType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ControlProcessType

Description: RAPS_ControlProcessType.jpg

Description: This enumeration models the the allowed values for the R-APS block logic in interconnected rings.
																																								An example showing the usage of these values is attached.

Contains Enumeration Literals:

- MAJOR_RING_NOT_INTERCONNECTED

- MAJOR_RING_INTERCONNECTED

- SUB_RING_NOT_INTERCONNECTED

- SUB_RING_INTERCONNECTED_WITH_VIRTUAL_CHANNEL

- SUB_RING_INTERCONNECTED_WITHOUT_VIRTUAL_CHANNEL

#### CreateCrossConnectionData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::CreateCrossConnectionData

Table 1 Attributes for Data Type CreateCrossConnectionData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| crossConnectionName | ObjectIdentifier | 1 | This attribute provides the name of the CrossConnection to be created. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| addressTableInfo | AddressTableInfo | 1 | This parameter provides the configuration information creating an AddressTable to be used by the CrossConnection or a reference to an existing AddressTable (in the Shared VLAN case) to be used by the CrossConnection. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portRefList | ObjectIdentifier | 2..* | This attribute provides the list of TPs/FPs/Ports to be connected by the new cross connection. TBD in G.8021 (Am2) | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portGroupList | PortGroup | 0..* | This parameter provides a list of port groups together with their contained ports. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| groupDefaultMode? | GroupDefaultMode | 1 | This attribute defines the default group filtering behavior as specified in IEEE 802.1Q section 8.8.6. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### CsfConfig enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::CsfConfig

Contains Enumeration Literals:

- DISABLED

  - This literal covers the following states of the CSF related MI informations:
																																								- MI_CSF_Enable is false
																																								- MI_CSFrdifdi_Enable is false
																																								- MI_CSFdci_Enable is false.

- ENABLED

  - This literal covers the following states of the CSF related MI informations:
																																								- MI_CSF_Enable is true
																																								- MI_CSFrdifdi_Enable is false
																																								- MI_CSFdci_Enable is false.

- ENABLED_WITH_RDI_FDI

  - This literal covers the following states of the CSF related MI informations:
																																								- MI_CSF_Enable is true
																																								- MI_CSFrdifdi_Enable is true
																																								- MI_CSFdci_Enable is false.

- ENABLED_WITH_RDI_FDI_DCI

  - This literal covers the following states of the CSF related MI informations:
																																								- MI_CSF_Enable is true
																																								- MI_CSFrdifdi_Enable is true
																																								- MI_CSFdci_Enable is true.

- ENABLED_WITH_DCI

  - This literal covers the following states of the CSF related MI informations:
																																								- MI_CSF_Enable is true
																																								- MI_CSFrdifdi_Enable is false
																																								- MI_CSFdci_Enable is true.

#### CtpAggregatorCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::CtpAggregatorCreateData

Description: This data type contains the configuration parameters for creating an Aggregator object accociated to a CTP.

Table 1 Attributes for Data Type CtpAggregatorCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| ctpRef | ETH_ConnectionTerminationPoint | 1 | This attribute provides the CTP identifier to be added to the LAG. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ctpAggregatorSourceCreateData | CtpAggregatorSourceCreateData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| ctpAggregatorSinkCreateData | CtpAggregatorSinkCreateData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### CtpAggregatorSinkCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::CtpAggregatorSinkCreateData

Description: This data type contains the configuration parameters for creating an Aggregator Sink object.

Table 1 Attributes for Data Type CtpAggregatorSinkCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pllThr | Integer | 1 | This attribute provisions the threshold for the number of active ports. If the number of active ports is more than zero but less than the provisioned threshold, a cPLL (Partial Link Loss) is raised. See section 9.7.1.2 of G.8021. Note: cannot find this in 802.1AX# | OpenModelAttribute - valueRange: 0..number of ports - support: MANDATORY - condition: none |
| isSsfReported | Boolean | 1 | This attribute provisions whether the SSF defect should be reported as fault cause or not. It models the ETH-LAG_FT_Sk_MI_SSF_Reported defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| filterConfig | ControlFrameFilter | 1 | This attribute models the FilterConfig MI defined in section 8.3/G.8021. It indicates the configured filter action for each of the 33 group MAC addresses for control frames. The 33 MAC addresses are: - All bridges address: 01-80-C2-00-00-10, - Reserved addresses: 01-80-C2-00-00-00 to 01-80-C2-00-00-0F, - GARP Application addresses: 01-80-C2-00-00-20 to 01-80-C2-00-00-2F. The filter action is Pass or Block. If the destination address of the incoming ETH_CI_D matches one of the above addresses, the filter process shall perform the corresponding configured filter action. If none of the above addresses match, the ETH_CI_D is passed. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### CtpAggregatorSourceCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::CtpAggregatorSourceCreateData

Description: This data type contains the configuration parameters for creating an Aggregator Source object.

#### DmThresholdValues datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::DmThresholdValues

Description: This data type contains the threshold values for frame delay related 2-way measurements.

Table 1 Attributes for Data Type DmThresholdValues

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| granularityPeriod | TimePeriod | 1 | This attribute defines the granularity period of the current registers to which the thresholds are assigned. | OpenModelAttribute - valueRange: 24hr, 15min, 1 - 14 min - support: MANDATORY - condition: none |
| nearEndCrossThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the near end cross threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nearEndClearThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the near end clear threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| farEndCrossThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the far end cross threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| farEndClearThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the far end clear threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| bidirectionalCrossThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the bidirectional cross threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| BidirectionalClearThreshold | StatisticalDmPerformanceParameters | 1 | This attribute contains the bidirectional clear threshold values of the delay measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ETH_TtpProblemList enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ETH_TtpProblemList

Description: This enumeration defines the valid list of problems for the ETH TTP Sink.

Contains Enumeration Literals:

- SSF

  - Server Signal Fail

- LCK

  - Locked

- LOCi?

  - Loss Of Continuity

- MMG

  - Mismerge

- UNM

  - Unexpected MEP

- UNP

  - Unexpected Periodicity

- UNPri

  - Unexpected Priority

- UNL

  - Unpextected MEG Level

- DEG

  - Degraded

- RDI

  - Remote Defect Indication

#### ETY_PhyType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ETY_PhyType

Applied Stereotypes:

- Deprecated

Contains Enumeration Literals:

- OTHER

- UNKNOWN

- NONE

- 2BASE_TL

- 10MBIT/S

- 10PASS_TS

- 100BASE_T4

- 100BASE_X

- 100BASE_T2

- 1000BASE_X

- 1000BASE_T

- 10GBASE-X

- 10GBASE_R

- 10GBASE_W

#### ETY_TtpProblemList enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ETY_TtpProblemList

Description: This enumeration defines the valid list of problems for the ETH TTP Sink.

Applied Stereotypes:

- Deprecated

Contains Enumeration Literals:

- LOC

  - Loss Of Continuity

- RDI

  - Remote Defect Indication

- FDI

  - Forward Defect Indication

#### ExpectedDefectMessage datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ExpectedDefectMessage

Description: This models the expected defect message.

Table 1 Attributes for Data Type ExpectedDefectMessage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepIdentifier | Integer | 1 | This attribute provides the identifier for the peer MEP. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| duration | Integer | 1 | This attribute reports on the duration of the expected defect message. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### FrameType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::FrameType

Contains Enumeration Literals:

- ADMIT_ONLY_VLAN_TAGGED_FRAMES

- ADMIT_ONLY_UNTAGGED_AND_PRIORITY_TAGGED_FRAMES

- ADMIT_ALL_FRAMES

#### GranularityPeriod enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::GranularityPeriod

Description: This enumeration defines the granularity periods of the PM registers at the pro-active measurements.

Contains Enumeration Literals:

- 24_HOUR

- 15_MINUTE

- Less_Than_15_MINUTE

#### GroupDefaultMode enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::GroupDefaultMode

Description: This enumeration provides the default group filtering behavior as specified in IEEE 802.1Q section 8.8.6. I.e., it defines how multicast MAC addresses are processed.

Contains Enumeration Literals:

- FORWARD_ALL_GROUPS

- FORWARD_UNREGISTERED_GROUPS

- FILTER_UNREGISTERED_GROUPS

#### LinkTraceResult datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::LinkTraceResult

Description: This data type contains the result from an individual LTR frame.

Table 1 Attributes for Data Type LinkTraceResult

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sourceAddress | MacAddress | 1 | This attribute contains the source MAC Address of an individual LTR frame result. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| timeToLiveValue | Integer | 1 | This attribute contains the Time To Live (TTL) value of an individual LTR frame result. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| dataTlvLength | Integer | 1 | This attribute contains the length (in number of octets) of the Data TLV of an individual LTR frame result. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### LmThresholdValues datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::LmThresholdValues

Description: This data type contains the threshold values for frame loss related 2-way measurements.

Table 1 Attributes for Data Type LmThresholdValues

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| granularityPeriod | TimePeriod | 1 | This attribute defines the granularity period of the current registers to which the thresholds are assigned. | OpenModelAttribute - valueRange: 24hr, 15min, 1 - 14 min - support: MANDATORY - condition: none |
| nearEndCrossThreshold | StatisticalLmPerformanceParameters | 1 | This attribute contains the near end cross threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| nearEndClearThreshold | StatisticalLmPerformanceParameters | 1 | This attribute is only valid for frame loss ratio parameters and counter type parameters working in the "standing condition method" (see G.7710, section 10.1.7.2: Threshold reporting) and contains the near end clear threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| farEndCrossThreshold | StatisticalLmPerformanceParameters | 1 | This attribute contains the far end cross threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| farEndClearThreshold | StatisticalLmPerformanceParameters | 1 | This attribute is only valid for frame loss ratio parameters and counter type parameters working in the "standing condition method" (see G.7710, section 10.1.7.2: Threshold reporting) and contains the far end clear threshold values of the loss measurements. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| bidirectionalUasCrossThreshold | Integer | 1 | This attribute contains the bidirectional cross threshold value of the UAS loss measurement. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| bidirectionalUasClearThreshold | Integer | 1 | This attribute is only valid for the UAS parameter working in the "standing condition method" (see G.7710, section 10.1.7.2: Threshold reporting) and contains the bidirectional clear threshold value of the UAS loss measurement. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### MacAddress primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MacAddress

Description: This primitive data type contains an Ethernet MAC address defined by IEEE 802a. The format of the address consists of 12 hexadecimal characters, grouped in pairs and separated by "-" (e.g., 03-27-AC-75-3E-1D).

#### MacOrMpId datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MacOrMpId

Description: This data type models the choice between a MAC address format and a Maintenance Point identifier format.

Table 1 Attributes for Data Type MacOrMpId

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mpIdentifier | MpIdentifier | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| macAddress | MacAddress | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### MepSetByCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSetByCreateData

Description: This data type contains the common parameters necessary for creating MEPs.

Table 1 Attributes for Data Type MepSetByCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. The default MEL is by agreement. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### MepSetData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSetData

Description: This data type contains the common parameters necessary for creating and modifying MEPs.

Table 1 Attributes for Data Type MepSetData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values: 3.33 ms for PS, 100 ms for PM, 1 s for FM | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MepSinkCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSinkCreateData

Description: This data type contains the parameters necessary for creating MEP Sinks.

Table 1 Attributes for Data Type MepSinkCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. The default MEL is by agreement. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| peerMepRefList | Integer | 0..* | This attribute models the MI_PeerMEP_ID[i] signal defined in G.8021 and configured as specified in G.8051. It provides the identifiers of the MEPs which are peer to the subject MEP. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| aisPeriod | OamPeriod | 1 | This attribute models the MI_AIS_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the AIS messages should be sent. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| aisPriority | Integer | 1 | This attribute models the MI_AIS_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the AIS messages should be sent. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCsfReported | Boolean | 1 | This attribute models the MI_CSF_Reported signal defined in G.8021 and configured as specified in G8051. It configures whether the secondary failure CSF should be reported or not. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCsfRdiFdiEnabled | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList |  | 1 | This attribute indicates the active alarms (i.e. current problems) of the MEP. | Inherited from Data Type MepSinkModifyData OpenModelAttribute - valueRange: cLOC[i], cUNL, cMMG, cUNM, cDEG, cUNP, cUNPr, cRDI, cSSF, cLCK - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values: 3.33 ms for PS, 100 ms for PM, 1 s for FM | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MepSinkModifyData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSinkModifyData

Description: This data type contains the parameters necessary for modifying MEP Sinks.

Table 1 Attributes for Data Type MepSinkModifyData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| peerMepRefList | Integer | 0..* | This attribute models the MI_PeerMEP_ID[i] signal defined in G.8021 and configured as specified in G.8051. It provides the identifiers of the MEPs which are peer to the subject MEP. | OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| aisPeriod | OamPeriod | 1 | This attribute models the MI_AIS_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the AIS messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| aisPriority | Integer | 1 | This attribute models the MI_AIS_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the AIS messages should be sent. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCsfReported | Boolean | 1 | This attribute models the MI_CSF_Reported signal defined in G.8021 and configured as specified in G8051. It configures whether the secondary failure CSF should be reported or not. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| isCsfRdiFdiEnabled | Boolean | 1 | This attribute models the MI_CSFrdifdiEnable signal defined in G.8021 and configured as specified in G8051. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| currentProblemList |  | 1 | This attribute indicates the active alarms (i.e. current problems) of the MEP. | OpenModelAttribute - valueRange: cLOC[i], cUNL, cMMG, cUNM, cDEG, cUNP, cUNPr, cRDI, cSSF, cLCK - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values: 3.33 ms for PS, 100 ms for PM, 1 s for FM | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MepSourceCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSourceCreateData

Description: This data type contains the parameters necessary for creating MEP Sources.

Table 1 Attributes for Data Type MepSourceCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| mepIdentifier | Integer | 1 | This attribute contains the identifier of the MEP. | OpenModelAttribute - valueRange: 0..8191 - support: MANDATORY - condition: none |
| mepMac | MacAddress | 1 | This attribute contains the MAC Address of the MEP. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| mel | Integer | 1 | This attribute identifies the MEG Level associated to the MEP. The default MEL is by agreement. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| megIdentifier | String | 1 | This attribute identifies the MEG associated to the MEP. | Inherited from Data Type MepSetByCreateData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| csfConfig | CsfConfig | 1 | This attribute models the combination of all CSF related MI signals (MI_CSF_Enable, MI_CSFrdifdi_Enable, MI_CSFdci_Enable) as defined in G.8021. | Inherited from Data Type MepSourceModifyData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| csfPeriod | OamPeriod | 1 | This attribute models the MI_CSF_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CSF messages should be sent. | Inherited from Data Type MepSourceModifyData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| csfPriority | Integer | 1 | This attribute models the MI_CSF_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CSF messages should be sent | Inherited from Data Type MepSourceModifyData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| apsPriority | Integer | 1 | This attribute specifies the priority of the APS messages. See section 8.1.5 APS insert process in G.8021. | Inherited from Data Type MepSourceModifyData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values: 3.33 ms for PS, 100 ms for PM, 1 s for FM | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MepSourceModifyData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MepSourceModifyData

Description: This data type contains the parameters necessary for modifying MEP Sources.

Table 1 Attributes for Data Type MepSourceModifyData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| csfConfig | CsfConfig | 1 | This attribute models the combination of all CSF related MI signals (MI_CSF_Enable, MI_CSFrdifdi_Enable, MI_CSFdci_Enable) as defined in G.8021. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| csfPeriod | OamPeriod | 1 | This attribute models the MI_CSF_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CSF messages should be sent. | OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| csfPriority | Integer | 1 | This attribute models the MI_CSF_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CSF messages should be sent | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| apsPriority | Integer | 1 | This attribute specifies the priority of the APS messages. See section 8.1.5 APS insert process in G.8021. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| adminState | AdminState | 1 | This attribute models the MI_Admin_State information defined in G.8021. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| clientMel | Integer | 1 | This attribute models the MI_Client_MEL signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| isCcEnabled | Boolean | 1 | This attribute models the MI_CC_Enable signal defined in G.8021 and configured as specified in G8051. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPeriod | OamPeriod | 1 | This attribute models the MI_CC_Period signal defined in G.8021 and configured as specified in G8051. It is the period at which the CCM message should be sent. Default values: 3.33 ms for PS, 100 ms for PM, 1 s for FM | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ccPriority | Integer | 1 | This attribute models the MI_CC_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the CCM message should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| lckPeriod | OamPeriod | 1 | This attribute models the MI_LCK_Period signal defined in G.8021 and configured as specified in G8051. It is the frequency at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 1s, 1min - support: MANDATORY - condition: none |
| lckPriority | Integer | 1 | This attribute models the MI_LCK_Pri signal defined in G.8021 and configured as specified in G8051. It is the priority at which the LCK messages should be sent. | Inherited from Data Type MepSetData OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### MessagePeriod enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MessagePeriod

Description: This enumeration defines the allowed values for the message period in on-demand measurements.
																																								Notes:
																																								The value 10ms is only used in synthetic loss measurements.
																																								The value 0 means that the value is not relevant.

Contains Enumeration Literals:

- 10MS

- 100MS

- 1S

- 10S

- 0

#### ModifyCrossConnectionData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ModifyCrossConnectionData

Table 1 Attributes for Data Type ModifyCrossConnectionData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| addressTableInfo | AddressTableInfo | 1 | This parameter provides the configuration information creating an AddressTable to be used by the CrossConnection or a reference to an existing AddressTable (in the Shared VLAN case) to be used by the CrossConnection. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portRefList | ObjectIdentifier | 2..* | This attribute provides the list of TPs/FPs/Ports to be connected by the new cross connection. TBD in G.8021 (Am2) | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portGroupList | PortGroup | 0..* | This parameter provides a list of port groups together with their contained ports. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| groupDefaultMode? | GroupDefaultMode | 1 | This attribute defines the default group filtering behavior as specified in IEEE 802.1Q section 8.8.6. | Inherited from Data Type SetCrossConnectionData OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### MpIdentifier primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::MpIdentifier

Description: This primitive data type contains a Maintenance Point (MEP or MIP) identifier. The format of the identifier is a 2-octet field where the 13 least significant bits are used to identify the Maintenance Point. The identifier is unique within the MEG.

#### OamPduGenerationType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OamPduGenerationType

Description: This enumeration defines the generation pattern of the on-demand OAM PDUs (messages).

Contains Enumeration Literals:

- SINGLE_INSTANCE

- REPETITIVE_INSTANCE

- SINGLE_SERIES

- REPETITIVE_SERIES

#### OamPeriod enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OamPeriod

Description: Provides the frequency for the OAM PDU insertion.

Contains Enumeration Literals:

- 3.33MS

  - Default for protection.

- 10MS

- 100MS

- 1S

- 10S

- 1MIN

- 10MIN

#### ObjectType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ObjectType

Description: This enumeration identifies the kind of object.

Contains Enumeration Literals:

- SINK

- SOURCE

- BIDIRECTIONAL

#### OnDemand1DmPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemand1DmPerformanceData

Description: This data type contains the near end on-demand frame delay PM data.

Table 1 Attributes for Data Type OnDemand1DmPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nearEndFrameDelayPerformanceParameters | OnDemandFrameDelayPerformanceParameters | 1 | This attribute contains the results of an on-demand frame delay measurement job in the ingress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OnDemand1SlPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemand1SlPerformanceData

Description: This data type contains the near end on-demand synthetic frame loss PM data.

Table 1 Attributes for Data Type OnDemand1SlPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nearEndFrameLossPerformanceParameters | OnDemandFrameLossPerformanceParameters | 1 | This attribute contains the results of an on-demand synthetic loss measurement job in the ingress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OnDemandFrameDelayPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandFrameDelayPerformanceData

Description: This data type contains the near end on-demand frame delay PM data.

Table 1 Attributes for Data Type OnDemandFrameDelayPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nearEndFrameDelayPerformanceParameters | OnDemandFrameDelayPerformanceParameters | 1 | This attribute contains the results of an on-demand frame delay measurement job in the ingress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| farEndFrameDelayPerformanceParameters | OnDemandFrameDelayPerformanceParameters | 1 | This attribute contains the results of an on-demand frame delay measurement job in the ingress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OnDemandFrameDelayPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandFrameDelayPerformanceParameters

Description: This data type contains the results of an on-demand delay measurement job.

Table 1 Attributes for Data Type OnDemandFrameDelayPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numberOfSamples | Integer | 1 | This attribute contains the number of received DM frames (successful samples) used for this frame delay measurement. | OpenModelAttribute - valueRange: non-negative - support: MANDATORY - condition: none |
| frameDelayList | Integer | 0..* | This attribute contains the frame delays measured in ns (nano second, 1x10e-9 seconds). The multiplicity is defined by the numberOfSamples attribute. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| frameDelayVariationList | Integer | 0..* | This attribute contains the frame delay variations measured in ns (nano second). The multiplicity is defined by (numberOfSamples - 1, for numberOfSamples > 0). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### OnDemandFrameLossPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandFrameLossPerformanceData

Description: This data type contains the near and far end on-demand frame loss PM data.

Table 1 Attributes for Data Type OnDemandFrameLossPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nearEndFrameLossPerformanceParameters | OnDemandFrameLossPerformanceParameters | 1 | This attribute contains the results of an on-demand synthetic loss measurement job in the ingress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| farEndFrameLossPerformanceParameters | OnDemandFrameLossPerformanceParameters | 1 | This attribute contains the results of an on-demand synthetic loss measurement job in the egress direction. | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OnDemandFrameLossPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandFrameLossPerformanceParameters

Description: This data type contains the results of an on-demand loss measurement job.

Table 1 Attributes for Data Type OnDemandFrameLossPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| totalTransmittedFrames | Integer | 1 | This attribute contains the total number of frames transmitted. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| totalLostFrames | Integer | 1 | This attribute contains the total number of frames lost. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| totalFrameLossRatio | Real | 1 | This attribute contains the frame loss ratio (number of lost frames divided by the number of total frames (N_LF / N_TF)). The accuracy of the value is for further study. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### OnDemandPduGenerationScheduleData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandPduGenerationScheduleData

Description: This data type contains the parameters necessary for the configuration of the PDU generation cycle.
																																								Not all parameters are always necessary; this is dependent on the PDU generation type (single instance, repetitive instance, single series, repetitive series).
																																								Single instance:
																																								- Start Time = any time
																																								- Stop Time = same as Start Time
																																								- Message Period = 0
																																								- Repetition Period = 0
																																								- Measurement Interval = 0
																																								Repetitive instance:
																																								- Start Time = any time
																																								- Stop Time  greater than start time
																																								- Message Period = 0
																																								- Measurement Interval = 0
																																								- Repetition Period = value required
																																								Single series:
																																								- Start Time = any time
																																								- Stop Time = greater than Start Time
																																								- Message Period = value required
																																								- Measurement Interval = Stop Time - Start Time
																																								- Repetition Period = 0
																																								Repetitive series:
																																								- Start Time = any time
																																								- Stop Time = greater than Start Time
																																								- Message Period = value required
																																								- Measurement Interval = value required
																																								- Repetition Period = value required {< Measurement Interval}

Table 1 Attributes for Data Type OnDemandPduGenerationScheduleData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| startTime | GeneralizedTime | 1 | This attribute contains the start date and time of the on-demand session. The default value of this attribute shall be the current time of the NE's real-time clock at the moment the measurement job is being created. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| stopTime | GeneralizedTime | 1 | This attribute contains the stop date and time of the on-demand session. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| oamPduGenerationType | OamPduGenerationType | 1 | This attribute contains the pattern that is used for the generation of OAM PDUs. | OpenModelAttribute - valueRange: See associated Enum. - support: MANDATORY - condition: none |
| messagePeriod | MessagePeriod | 1 | This attribute contains the frequency of the OAM message (PDU) generation within a series. Note that the value 0 means that only one OAM message per measurement interval is generated. Note that for on-demand LM, practical period should be 1 sec or longer.# | OpenModelAttribute - valueRange: See corresponding Enum - support: MANDATORY - condition: none |
| measurementInterval | Integer | 1 | This attribute contains the discrete non overlapping periods of time during which measurements are performed (i.e., OAM messages are generated) and reports are gathered at the end of the measurement intervals. Note that the value 0 means a degenerated measurement interval with a single OAM message and the report is sent as immediately as possible. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| repetitionPeriod | RepetitionPeriod | 1 | This attribute contains the time between the start of two measurement intervals. This IS applicable for the repetitive instance type and MAY be applicable for the repetitive series type. Note that a value of 0 means not applicable (NA), which is for the cases of single instance, single series, or repetitive series without extra gap in between the measurement intervals (i.e., also as known as continuous series). | OpenModelAttribute - valueRange: See corresponding Enum - support: MANDATORY - condition: none |

#### OnDemandPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandPerformanceData

Description: This data type choice is used as the generic data type for all kinds of 1 way on-demand PM data.

Applied Stereotypes:

- Choice

Table 1 Attributes for Data Type OnDemandPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _onDemandFrameFelayPerformanceData | OnDemandFrameDelayPerformanceData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemandFrameLossPerformanceData | OnDemandFrameLossPerformanceData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OnDemandSinkPerformanceData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OnDemandSinkPerformanceData

Description: This data type choice is used as the generic data type for all kinds of 1 way on-demand PM data.

Applied Stereotypes:

- Choice

Table 1 Attributes for Data Type OnDemandSinkPerformanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| _onDemand1SlPerformanceData | OnDemand1SlPerformanceData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| _onDemand1DmPerformanceData | OnDemand1DmPerformanceData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### OperationType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::OperationType

Contains Enumeration Literals:

- REVERTIVE

- NON_REVERTIVE

#### PcpCoding enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::PcpCoding

Description: This enum models the coding of the Priority Code Point as defined in section "Priority Code Point encoding" of IEEE 802.1Q.

Contains Enumeration Literals:

- 8P0D

- 7P1D

- 6P2D

- 5P3D

- DEI

  - This enumeration value means that all priorities should be drop eligible.
																																								DEI = Drop Eligibility Indicator

#### PortGroup datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::PortGroup

Description: This data type provides the list of TPs/FPs associated to a port group.

Table 1 Attributes for Data Type PortGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| terminationPointList | ObjectIdentifier | 2..* |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### PriorityConfiguration datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::PriorityConfiguration

Table 1 Attributes for Data Type PriorityConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| priority | Integer | 1 |  | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| queueId | Integer | 1 |  | OpenModelAttribute - valueRange: 1..8 - support: MANDATORY - condition: none |

#### PriorityMapping datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::PriorityMapping

Description: This data type provides the priority mapping done in the "P Regenerate" process defined in G.8021.

Table 1 Attributes for Data Type PriorityMapping

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| Priority0 | Integer | 1 | This attribute defines the new priority value for the old priority value 0. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority1 | Integer | 1 | This attribute defines the new priority value for the old priority value 1. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority2 | Integer | 1 | This attribute defines the new priority value for the old priority value 2. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority3 | Integer | 1 | This attribute defines the new priority value for the old priority value 3. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority4 | Integer | 1 | This attribute defines the new priority value for the old priority value 4. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority5 | Integer | 1 | This attribute defines the new priority value for the old priority value 5. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority6 | Integer | 1 | This attribute defines the new priority value for the old priority value 6. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |
| Priority7 | Integer | 1 | This attribute defines the new priority value for the old priority value 7. | OpenModelAttribute - valueRange: 0..7 - support: MANDATORY - condition: none |

#### ProActive1DmPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProActive1DmPerformanceParameters

Description: This data type contains the near end pro-active delay measurement PM data.

Table 1 Attributes for Data Type ProActive1DmPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| statisticalNearEndFrameDelayPerformanceParameters | StatisticalDmPerformanceParameters | 1 | This attribute contains the statistical near end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProActive1SlPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProActive1SlPerformanceParameters

Description: This data type contains the near end pro-active PM data.

Table 1 Attributes for Data Type ProActive1SlPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| statisticalNearEndFrameLossPerformanceParameters | StatisticalLmPerformanceParameters | 1 | This attribute contains the statistical near end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProActiveDmPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProActiveDmPerformanceParameters

Description: This data type contains the near end, far end and bidirectional pro-active delay measurement PM data.

Table 1 Attributes for Data Type ProActiveDmPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| statisticalNearEndFrameDelayPerformanceParameters | StatisticalDmPerformanceParameters | 1 | This attribute contains the statistical near end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| statisticalFarEndFrameDelayPerformanceParameters | StatisticalDmPerformanceParameters | 1 | This attribute contains the statistical far end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| statisticalBidirectionalFrameDelayPerformanceParameters | StatisticalDmPerformanceParameters | 1 | This attribute contains the statistical bidirectional performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### ProActiveLmPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProActiveLmPerformanceParameters

Description: This data type contains the near and far end pro-active loss measurement PM data.

Table 1 Attributes for Data Type ProActiveLmPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| statisticalNearEndFrameLossPerformanceParameters | StatisticalLmPerformanceParameters | 1 | This attribute contains the statistical near end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| statisticalFarEndFrameLossPerformanceParameters | StatisticalLmPerformanceParameters | 1 | This attribute contains the statistical far end performnace parameters. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| bidirectionalUas | Integer | 1 | This attribute contains the bidirectional UAS (unavailable seconds) detected in the monitoring interval. | OpenModelAttribute - valueRange: 0..900 for 15min interval or 0..86400 for 24 hr interval. - support: MANDATORY - condition: none |

#### ProActiveOamType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProActiveOamType

Contains Enumeration Literals:

- LM_CCM

- LM_LMM

- 1SL

- SL

- 1DM

- DM

#### ProtectionBridgeType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::ProtectionBridgeType

Description: This enumeration models the valid configurations of the Protection Type and Bridge Type defined in Table 11-2 of G.8031 (06/2011).

Contains Enumeration Literals:

- 1_PLUS_1_UNIDIRECTIONAL_NO_APS

- 1_PLUS_1_UNIDIRECTIONAL_WITH_APS

- 1_PLUS_1_BIDIRECTIONAL_WITH_APS

- 1_FOR_1_BIDIRECTIONAL_WITH_APS_USING_SELECTOR_BRIDGE

- 1_FOR_1_BIDIRECTIONAL_WITH_APS_USING_BROADCAST_BRIDGE

#### QueueConfiguration datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::QueueConfiguration

Table 1 Attributes for Data Type QueueConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| queueId | Integer | 1 | This attribute indicates the queue id. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| queueDepth | Integer | 1 | This attribute defines the depth of the queue in bytes. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| queueThreshold | Integer | 1 | This attribute defines the threshold of the queue in bytes. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### RepetitionPeriod enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::RepetitionPeriod

Description: This enumeration defines the allowed values for the repetition period in on-demand measurements.
																																								Note: The value 0 means that the value is not relevant.

Contains Enumeration Literals:

- 1MIN

- 1S

- 10S

- 0

#### RingPortRef datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::RingPortRef

Description: This data type identifies an ETH CTP and its role in the RPL (i.e. either owner port, neighbour port, or normal port).

Table 1 Attributes for Data Type RingPortRef

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| ctpRef | ETH_ConnectionTerminationPointBidirectional | 1 | This attribute identifies an ETH CTP. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| rplPort | RplPort | 1 | This attribute identifies the role of the ETH CTP wrt the RPL. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |

#### RplPort enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::RplPort

Description: This enumeration identifies the kind of port wrt an RPL.

Contains Enumeration Literals:

- NEIGHBOUR_PORT

- OWNER_PORT

- NORMAL_PORT

#### SchedulingConfiguration datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::SchedulingConfiguration

Description: The syntax of this dataType is pending on the specification in G.8021, which is for further study.

#### SetCrossConnectionData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::SetCrossConnectionData

Table 1 Attributes for Data Type SetCrossConnectionData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| addressTableInfo | AddressTableInfo | 1 | This parameter provides the configuration information creating an AddressTable to be used by the CrossConnection or a reference to an existing AddressTable (in the Shared VLAN case) to be used by the CrossConnection. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portRefList | ObjectIdentifier | 2..* | This attribute provides the list of TPs/FPs/Ports to be connected by the new cross connection. TBD in G.8021 (Am2) | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| portGroupList | PortGroup | 0..* | This parameter provides a list of port groups together with their contained ports. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| groupDefaultMode? | GroupDefaultMode | 1 | This attribute defines the default group filtering behavior as specified in IEEE 802.1Q section 8.8.6. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### SharedVlanAddressTableReference datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::SharedVlanAddressTableReference

Description: This data type contains the identifier of the address table to be used in the shared mode by the cross connection.

Table 1 Attributes for Data Type SharedVlanAddressTableReference

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| tableReference | ObjectIdentifier | 1 |  | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### SncpGroupState enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::SncpGroupState

Contains Enumeration Literals:

- NO-REQUEST

- LOCKOUT

- FORCED_SWITCH

- SIGNAL_FAIL

- MANUAL_SWITCH

- DO_NOT_REVERT

- EXERCISE

- REVERSE_REQUEST

#### StatisticalDmPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::StatisticalDmPerformanceParameters

Description: This data type contains the statistical delay measurement performance parameters.

Table 1 Attributes for Data Type StatisticalDmPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| minimumFrameDelay | Integer | 1 | This attribute contains the minimum frame delay observed over the monitored period. It is measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| averageFrameDelay | Integer | 1 | This attribute contains the average frame delay observed over the monitored period. It is measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| maximumFrameDelay | Integer | 1 | This attribute contains the maximum frame delay observed over the monitored period. It is measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| minimumFrameDelayVariation | Integer | 1 | This attribute contains the minimum frame delay variation measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| averageFrameDelayVariation | Integer | 1 | This attribute contains the average frame delay variation measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| maximumFrameDelayVariation | Integer | 1 | This attribute contains the maximum frame delay variation measured in units of ns (nano second, 1x10e-9 seconds). | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |

#### StatisticalLmPerformanceParameters datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::StatisticalLmPerformanceParameters

Description: This data type contains the statistical loss measurement performance parameters.

Table 1 Attributes for Data Type StatisticalLmPerformanceParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| minimumFrameLossRatio | Real | 1 | This attribute contains the minimum frame loss ratio calculated over a period of time. The accuracy of the value is for further study. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| averageFrameLossRatio | Real | 1 | This attribute contains the average frame loss ratio calculated over a period of time. The accuracy of the value is for further study. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| maximumFrameLossRatio | Real | 1 | This attribute contains the maximum frame loss ratio calculated over a period of time. The accuracy of the value is for further study. | OpenModelAttribute - valueRange: NA - support: MANDATORY - condition: none |
| ses | Integer | 1 | This attribute contains the SES detected in the monitoring interval. | OpenModelAttribute - valueRange: 0..900 for 15min interval or 0..86400 for 24 hr interval. - support: MANDATORY - condition: none |
| uas | Integer | 1 | This attribute contains UAS (unavailable seconds) detected in the monitoring interval. | OpenModelAttribute - valueRange: 0..900 for 15min interval or 0..86400 for 24 hr interval. - support: MANDATORY - condition: none |

#### SwitchDirection enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::SwitchDirection

Contains Enumeration Literals:

- SIGNAL_SELECTED_FROM_WORKING

- SIGNAL_SELECTED_FROM_PROTECTING

#### TimePeriod_tbdeleted enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::TimePeriod_tbdeleted

Contains Enumeration Literals:

- 1SEC

- 1MIN

#### TrafficConditioningConfiguration datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::TrafficConditioningConfiguration

Table 1 Attributes for Data Type TrafficConditioningConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cir | Integer | 1 | This attribute indicates the Committed Information Rate in bits/s. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| cbs | Integer | 1 | This attribute indicates the Committed Burst Size in bytes. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| eir | Integer | 1 | This attribute indicates the Excess Information Rate in bits/s. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ebs | Integer | 1 | This attribute indicates the Excess Burst Size in bytes. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| couplingFlag | Boolean | 1 | This attribute indicates the coupling flag. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| colourMode | ColourMode | 1 | This attribute indicates the colour mode. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| queueId | Integer | 1 | This attribute indicates the queue id. | OpenModelAttribute - valueRange: 1..8 - support: MANDATORY - condition: none |

#### TtpAggregationPortCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::TtpAggregationPortCreateData

Description: This data type contains the configuration parameters for creating an Aggregation Port object accociated to a TTP.

Table 1 Attributes for Data Type TtpAggregationPortCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| ttpRef | ETH_TrailTerminationPoint | 1 | This attribute provides the TTP identifier to be added to the LAG. | OpenModelAttribute - valueRange: See data type - support: MANDATORY - condition: none |
| ttpAggregationPortSourceCreateData | TtpAggregationPortSourceCreateData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |
| ttpAggregationPortSinkCreateData | TtpAggregationPortSinkCreateData | 0..1 |  | OpenModelAttribute - valueRange: no range constraint - support: MANDATORY - condition: none |

#### TtpAggregationPortSinkCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::TtpAggregationPortSinkCreateData

Description: This data type contains the configuration parameters for creating an Aggregation Port Sink object.

#### TtpAggregationPortSourceCreateData datatype

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::TtpAggregationPortSourceCreateData

Description: This data type contains the configuration parameters for creating an Aggregation Port Source object.

Table 1 Attributes for Data Type TtpAggregationPortSourceCreateData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| actorAdminState | BitString | 1 | See IEEE 802.1AX: A string of 8 bits, corresponding to the administrative values of Actor_State (5.4.2) as transmitted by the Actor in LACPDUs. The first bit corresponds to bit 0 of Actor_State (LACP_Activity), the second bit corresponds to bit 1 (LACP_Timeout), the third bit corresponds to bit 2 (Aggregation), the fourth bit corresponds to bit 3 (Synchronization), the fifth bit corresponds to bit 4 (Collecting), the sixth bit corresponds to bit 5 (Distributing), the seventh bit corresponds to bit 6 (Defaulted), and the eighth bit corresponds to bit 7 (Expired). These values allow administrative control over the values of LACP_Activity, LACP_Timeout, and Aggregation. This attribute value is read-write. | OpenModelAttribute - valueRange: SIZE(1..8) - support: MANDATORY - condition: none |
| isTxPauseEnabled | Boolean | 1 | This attribute identifies whether the Transmit Pause process is enabled or not. It models the MI_TxPauseEnable defined in G.8021. | OpenModelAttribute - valueRange: true, false - support: MANDATORY - condition: none |

#### Vid primitive type

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::Vid

Description: This primitive type models the 12 Bit VLAN identifier of a VLAN tag.

#### VlanType enumeration

Qualified Name: G.8052_v2.07-model::Type Definitions Specific::VlanType

Description: This enumeration contains the Ethertypes defined in IEEE 802.1Q.

Contains Enumeration Literals:

- C_Tag

  - 0x8100

- S_Tag

  - 0x88a8

- I_Tag

  - 88-e7

### Associations